C-Kermit for OS/2

Version 1.00

by Chris Adie et al.

Copyright (C) 1988 Edinburgh University Computing Service.

Portions of this document are Copyright (C) 1981, 1988 Trustees

of Columbia University in the city of New York.

Permission is granted to any individual or institution to use, copy,

or redistribute this document so long as it is not sold for

profit, and provided this copyright notice is retained.

1 INTRODUCTION

1.1 Acknowledgements

C-Kermit was written  by Frank da  Cruz, Bill Catchings,  Jeff Damens and  Herm

Fischer, with  contributions by  many others.   It was  originally written  for

computers running the Unix operating system, but it has been ported to the  VAX

running VMS, the Apple  Macintosh and the Commodore  Amiga, among others.   The

program was  ported  to OS/2  by  Chris Adie  (Edinburgh  University  Computing

Service).

This documentation is based closely on the C-Kermit manual by Frank da Cruz and

Herm Fischer.  The section on  the OS/2 file system  is based on the  MS-Kermit

manual, by Christine Gianone, Frank da Cruz and Joe Doupnik.  It should be read

in conjunction with a  more general description of  Kermit such as "The  Kermit

File Transfer Protocol" by Frank da  Cruz (Digital Press, ISBN  0-932376-88-6),

or the Kermit User Guide, available in machine readable form from your  nearest

Kermit distribution centre.

1.2 OS/2 Kermit Capabilities

C-Kermit provides traditional  (Unix-style) command line  operation as well  as

interactive command prompting and execution.  The command line options  provide

access to a basic  subset of C-Kermit's  capabilities; the interactive  command

set is far richer.

C-Kermit is a protected-mode program.  It will not run in the DOS compatability

environment.  This means that it will continue running (eg transferring  files)

even when it is not the foreground session.

All numbers in the C-Kermit documentation are decimal unless noted otherwise.

Summary of capabilities:

Local operation:                   Yes

Remote operation:                  No

Login scripts:                     Yes (UUCP style)

Transfer text files:               Yes

Transfer binary files:             Yes

Wildcard send:                     Yes

File transfer interruption:        Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes

Communication settings:            Yes

Transmit BREAK:                    Yes

Support for dialout modems:        Yes

IBM mainframe communication:       Yes

Transaction logging:               Yes

Session logging:                   Yes

Debug logging:                     Yes (Debug logging is disabled.)

Packet logging:                    Yes

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Local file management:             Yes

Command/Init files:                Yes

Long packets:                      Yes

Sliding Windows:                   No

File attributes packets:           No

Command macros:                    No

Raw file transmit:                 No

1.3 Requirements

C-Kermit will run on a computer with  an 80286 or 80386 processor running  OS/2

version 1.0 or higher.  It runs in character mode - in other words it is not  a

Presentation Manager  application.   However, it  will  run in  a  Presentation

Manager window as a character application.  Normally, though, you would run  it

from the command processor (CMD.EXE) prompt.

1.4 The Serial Port

Naturally, a serial port (COM1 or COM2) is required.  The OS/2 Standard Edition

comms port device driver must be loaded using a line like one of the  following

in the CONFIG.SYS file:

DEVICE=COM01.SYS

or                  DEVICE=COM02.SYS

COM01.SYS is used for PC/AT - type machines, while for PS/2s COM02.SYS must  be

used.  C-Kermit  will  not work  if  this device  driver  is not  loaded.   (It

provides the Category 1 IOCTLs which are used extensively within the program.)

The connecting cable and the  modem (or other computer,  PAD etc to which  your

computer is connected) must satisfy the  requirements of your computer's  RS232

interface.  In particular, the computer will provide two output control signals

(RTS and DTR), and  may expect to  see signals on four  input lines (DCD,  DSR,

CTS, RI).  The precise behaviour of  these lines is software configurable  (for

instance by using the  OS/2 'MODE' command), and  C-Kermit makes no attempt  to

impose a particular method of using them.

By default, the DTR and RTS line will both go ON when C-Kermit opens the  comms

port, and they will go OFF when it is closed.

The default behaviour for  the input lines is  that DSR and CTS  must be ON  to

enable the port to work.   If the modem you are  connected to does not  provide

these signals, you can 'loop back' the  RTS output signal from the computer  to

DSR and CTS, using  a suitably modified  cable.  An alternative  is to use  the

MODE command to disable  the DSR and  CTS inputs.  To do  this, type a  command

similar to the following at the OS/2 CMD prompt:

MODE COM1:9600,N,8,1,OCTS=OFF,ODSR=OFF,IDSR=OFF

You can check the effect using:

MODE COM1

which reports  the current  settings  of COM1.   Note  that on  some  machines,

C-Kermit may appear  to work even  although DSR  and CTS are  not connected  to

anything, nor disabled using 'MODE'.  This  is because unconnected input  lines

tend to 'float high'.  Although this  situation may not cause any problems,  it

is not good practice - you should explicitly disable the inputs as above.

The 'MODE' utility also allows you to  change the baud rate, parity, number  of

data bits and number of stop  bits.  C-Kermit provides facilities for  changing

the baud rate and parity too (see later in this manual), but when it starts up,

it resets the parity to none and the number of data bits to 8.  Any changes  to

baud rate and parity will remain in effect after C-Kermit terminates.

If you change the parity within C-Kermit, it will ajust the number of data bits

to cope.  There is no way of changing the number of stop bits within  C-Kermit:

use 'MODE' to do this.

1.5 Emergency Exit

EMERGENCY EXIT: The Control-C key cannot be used to terminate C-Kermit.

The Control-Break key will  always work, but it  takes you back to  the

Program Selector rather than the CMD.EXE command interpreter from which

Kermit was invoked.

2 THE OS/2 FILE SYSTEM

The features of the OS/2 file system  of greatest interest to Kermit users  are

the form of the file specifications,  and the formats of the files  themselves.

Note that the following discussion refers to the MS-DOS compatible file  system

supported by  initial  versions of  OS/2.   Installable file  systems  are  not

covered here  - they  are  significantly different,  and  the extent  to  which

C-Kermit will work under such file  systems is unknown (because no  installable

file system has been released at the time of writing).

2.1 File Specifications

OS/2 file specifications are of the form

DEVICE:\PATHNAME\NAME.TYPE

where DEVICE stands for a single character identifier (for instance, A for  the

first floppy disk,  C for  the first fixed  disk, D  for a  RAM disk  emulator)

followed by a colon (':'), PATHNAME is up to 63 characters of identifier(s) (up

to 8 characters each) surrounded by backslashes ('\'), NAME is an identifier of

up to 8 characters, and TYPE is an identifier of up to 3 characters in  length.

Device and pathname may be omitted.  The first backslash in the pathname may be

omitted if the  specified path is  relative to the  current directory.  In  the

path field, '.' means the current directory, '..' means the parent directory.

Pathname is normally omitted,  but can be specified  in all C-Kermit  commands.

Device and  directory pathnames,  when omitted,  default to  either the  user's

current disk  and  directory,  or  to the  current  directory  search  path  as

specified in the PATH environment variable,  depending on the context in  which

the file name appears.

When this document says that a file  is searched for "in the current path,"  it

means that C-Kermit looks on the current  disk and directory first, and if  the

file is not found, then the directories listed in the PATH environment variable

are searched.  If the PATH environment variable is empty, Kermit looks only  at

the current disk and directory.

NAME.TYPE is sufficient to  specify a file on  the current disk and  directory,

and only this information is sent along  by C-Kermit with an outgoing file  (by

default).

The device, path, name, and type fields may contain uppercase letters,  digits,

and the special characters '-' (dash), '_' (underscore), '$' (dollar sign), '&'

(ampersand), '#'  (number sign),  '@' (at  sign), '!'  (exclamation mark),  '''

(single  quote),  '()'  (parentheses),  '{}'  (curly  braces),  '^'  (caret  or

circumflex), '~' (tilde), and '`' (accent grave).  Normally, you should confine

your filenames to letters and  digits for maximum transportability to  non-OS/2

systems (by default, C-Kermit will translate filenames being sent by converting

non-alphanumeric characters  to  'X').   When you  type  lowercase  letters  in

filenames, they  are  converted  automatically  to  uppercase.   There  are  no

imbedded or trailing spaces.  Other characters may not be included; there is no

mechanism for "quoting" otherwise illegal characters in filenames.  The  fields

of the  file specification  are set  off from  one another  by the  punctuation

indicated above (ie colon, backslash and dot).

The name field is the primary identifier  for the file.  The type, also  called

the extension or suffix, is an indicator which, by convention, tells what  kind

of file we have.  For instance FOO.BAS is  the source of a BASIC program  named

FOO; FOO.OBJ  might be  the  relocatable object  module produced  by  compiling

FOO.BAS; FOO.EXE could be  an executable program  produced by loading  FOO.OBJ,

and so forth.  .EXE is the normal suffix for executable programs.

OS/2 allows a group of files to be specified in a single file specification  by

including the special "wildcard"  characters, '*' and '?'.   A '*' matches  any

string of  characters  from the  current  position to  the  end of  the  field,

including no characters at all; a  '?' matches any single character.  Here  are

some examples:

*.BAS           All files  of type  BAS  (BASIC source  files) in  the  current

directory.

FOO.*           Files of all types with name FOO.

F*.*            All files whose names start with F.

*.?             All files with  types exactly one  character long,  or with  no

type at all.

Wildcard notation is used on many computer  systems in similar ways, and it  is

the mechanism most commonly used to instruct Kermit to send a group of files.

You should  bear  in mind  that  other  (non-OS/2) systems  may  use  different

wildcard characters.  For instance VMS and the DEC-20 use '%' instead of '?' as

the single character wildcard; when using  C-Kermit to request a wildcard  file

group from a Kermit-20 server, the OS/2 '?' must be replaced by the DEC-20 '%'.

2.2 File Formats

OS/2 systems  store  files  as  streams of  8-bit  bytes,  with  no  particular

distinction among  text, program  code,  and binary  files.  ASCII  text  files

consist of lines separated  by carriage-return-linefeed sequences (CRLFs),  and

this  conforms  exactly  to  the  way  Kermit  represents  text  files   during

transmission.

OS/2 (unlike CP/M) knows the exact end of a file because it keeps a byte  count

in the directory, so one would  expect no particular confusion in this  regard.

However, certain MS-DOS and OS/2 programs  continue to use the CP/M  convention

of terminating a text file with a Control-Z character.  This may cause problems

when the file is transferred elsewhere,  since other systems may object to  the

Control-Z. By default, therefore, C-Kermit treats the first Control-Z it  finds

in the  file  as  being  equivalent  to  end-of-file.   The  Control-Z  is  not

transmitted to  the other  system.   Of course,  this  leads to  problems  when

transferring non-text files, when we do want  any Control-Zs in the file to  be

sent.  To achieve  this, the  C-Kermit 'set file  type binary'  command may  be

used.  The opposite, 'set file type text', is the default.

Non-OS/2 systems may be confused by nonstandard ASCII files sent by C-Kermit:

- Files containing any  of the  8-bit "extended ASCII"  characters may  need

conversion (or translation) to 7-bit ASCII.

- Files produced by word processing programs  like Word Perfect or  Wordstar

may contain special binary formatting codes, and could need conversion  to

conventional 7-bit  ASCII format  prior  to transmission,  using  commonly

available "exporter" programs.

- Spreadsheet or  database  files  usually need  special  formatting  to  be

meaningful to non-OS/2 recipients (though they can be transmitted  between

OS/2 and MS-DOS systems with Kermit).

- BASIC programs  are normally  saved  in a  binary "tokenized"  form.   Use

BASIC's ",a" option to save them as regular ASCII text, as in

save"foofa",a

In general,  when attempting  to transfer  non-text files  between OS/2  and  a

different kind of system, consult the Kermit manual for that system.

3 FILE TRANSFER

When C-Kermit  is  transferring a  file,  the screen  (stdout)  is  continously

updated to show the progress of the file  transer.  A dot is printed for  every

four data packets, other packets are shown by type:

I Exchange Parameter Information

R Receive Initiate

S Send Initiate

F File Header

G Generic Server Command

C Remote Host Command

N Negative Acknowledgement (NAK)

E Fatal Error

T Indicates a timeout occurred

Q Indicates a damaged, undesired, or illegal packet was received

% Indicates a packet was retransmitted

You may type certain "interrupt" commands during file transfer:

Control-F:  Interrupt the current File, and go on to the next (if any).

Control-B:  Interrupt the entire Batch of files, terminate the transaction.

Control-R:  Resend the current packet.

Control-A:  Display a status report for the current transaction.

These  interrupt  characters  differ  from  the  ones  used  in  other   Kermit

implementations to  avoid  conflict with  commonly  used Unix  shell  interrupt

characters.  This reason, of course, is not  valid under OS/2, and in a  future

version of C-Kermit for OS/2 it is hoped to rectify this.

CAUTION: If Control-F or Control-B is used to cancel an incoming  file,

and a file of the same name previously existed, and the "file  warning"

feature is  not  enabled, then  the  previous  copy of  the  file  will

disappear.

The Emergency Exit key (Control-Break) can  be used to terminate Kermit in  the

middle of a file transfer.  This will not terminate the transfer gracefully (it

is a 'brute  force' method),  so the remote  Kermit system  may be  left in  an

undefined state.

4 COMMAND LINE OPERATION

The C-Kermit command line syntax conforms to the Proposed Syntax Standards  for

Unix System Commands put  forth by Kathy Hemenway  and Helene Armitage of  AT&T

Bell Laboratories in Unix/World, Vol.1,  No.3, 1984.  The implications of  this

for specifying command-line options are:

- An option name is a single character.

- Options are delimited by '-'.

- Options with no arguments may be grouped (bundled) behind one delimiter.

- Options which take an argument must not have the argument omitted.

- Arguments immediately follow options, separated by whitespace.

- The order of options does not matter.

- A '-' preceded and followed by whitespace means standard input.

- A group of bundled options may end with an option that has an argument.

The following notation is used in command descriptions:

fn      An  OS/2  file  specification,   possibly  containing  the   "wildcard"

characters '*' or '?'.

fn1     An OS/2 file specification which may not contain '*' or '?'.

rfn     A remote file specification  in the remote  system's own syntax,  which

may denote a single file or a group of files.

rfn1    A remote file specification which should denote only a single file.

n       A decimal number between 0 and 94.

c       A decimal number between 0 and  127 representing the value of an  ASCII

character.

cc      A decimal number between  0 and 31, or  else exactly 127,  representing

the value of an ASCII control character.

[ ]     Any field in square braces is optional.

{x,y,z} Alternatives are listed in curly braces.

C-Kermit command  line  options may  specify  any combination  of  actions  and

settings.  If  C-Kermit  is invoked  with  a  command line  that  specifies  no

actions, then it  will issue  a prompt  and begin  interactive dialog.   Action

options specify either protocol transactions or terminal connection.

4.1 Command Line Options

-s fn   Send the specified file.  If fn is '-' then kermit sends from  standard

input, which may come from a file:

kermit -s - < foo.bar

or a parallel process:

dir *.txt | kermit -s -

You cannot use this mechanism to  send console typein.  If you want  to

send a file whose  actual name is  '-' you can precede  it with a  path

name, as in

kermit -s .\-

The argument fn may contain wildcards.  For instance,

kermit -s ck*.h

will send all the files matching the specification 'ck*.h'.

-r      Receive a file or files.  Wait passively for files to arrive.

-k      Receive (passively) a file or files,  sending them to standard  output.

This option can be used in several ways:

kermit -k

Displays the incoming files on your screen.

kermit -k > fn1

Sends the incoming file  or files to the  named file, fn1.   If

more than one file arrives, all are concatenated together  into

the single file fn1.

kermit -k | command

Pipes the  incoming  data (single  or  multiple files)  to  the

indicated command, as in

kermit -k | sort > sorted.txt

-a fn1  If you have specified a file transfer option, you may give an alternate

name for a single file with the -a ("as") option.  For example,

kermit -s foo -a bar

sends the file foo telling the receiver that its name is bar.  If  more

than one file arrives or  is sent, only the  first file is affected  by

the -a option:

kermit -ra baz

stores the first incoming file under the name baz.

-x      Begin server operation.

Before proceeding, a few words about remote and local operation are  necessary.

Kermit (in general, not  just C-Kermit) is  "local" if it is  running on PC  or

workstation that you are  using directly, or  if it is  running on a  multiuser

system and transferring files over an  external communication line -- not  your

job's controlling terminal or console. Kermit is  remote if it is running on  a

multiuser system and  transferring files  over its  own controlling  terminal's

communication line, connected to your PC or workstation.

C-Kermit running under OS/2 is always used in local mode, with the serial  port

at  the  back  of  the  machine  designated  for  file  transfer  and  terminal

connection.  Which serial  port to  use is  determined by  the -l  command-line

option:

-l dev  Line -- Specify  a serial line  to use for  file transfer and  terminal

connection, as in

kermit -l com2

The default line is COM1.

There are a number of other options associated with the use of the serial port:

-b n    Baud -- Specify the baud rate for  the line given in the -l option,  as

in

kermit -l com2 -b 9600

It is  good practice  to include  this with  the -l  option, since  the

current speed of the serial port might not be what you expect.

-p x    Parity -- x may be one of e,o,m,s,n (even, odd, mark, space, or  none).

If parity is other than none, then the 8th-bit prefixing mechanism will

be used  for  transferring 8-bit  binary  data, provided  the  opposite

Kermit agrees.  The default parity is none.

-t      Specifies half  duplex,  line  turnaround with  XON  as  the  handshake

character.

The following command line options access a remote Kermit server:

-g rfn  Actively request a remote server to  send the named file or files;  rfn

is a file specification in the remote host's own syntax.

-f      Send a 'finish' command to a remote server.

The following command  line options  are to do  with connecting  to the  remote

system as a terminal:

-c      Establish  a  terminal  connection   over  the  specified  or   default

communication line, before any  protocol transaction takes place.   Get

back to  the local  system  by typing  the escape  character  (normally

Control-]) followed by the letter 'c'.

-n      Like -c, but after  a protocol transaction takes  place; -c and -n  may

both be used in the same command.  The use of -n and -c is  illustrated

below.

Several other command-line options are provided:

-i      Specifies that files should be sent or received exactly "as is" with no

conversions.  This option is  necessary for transmitting binary  files,

and is equivalent to the 'set file type binary' interactive command.

-w      Write-Protect -- Avoid filename collisions for incoming files.

-e n    Extended packet length -- Specify  that C-Kermit is allowed to  receive

packets up  to length  n, where  n may  be between  10 and  some  large

number, like 1000, depending on the system.  The default maximum length

for received packets is 90.  Packets  longer than 94 will be used  only

if the other  Kermit supports,  and agrees  to use,  the "long  packet"

protocol extension.

-q      Quiet -- Suppress screen update  during file transfer, for instance  to

allow a file transfer to proceed in the background.

-d      Debug --  Record debugging  information in  the file  debug.log in  the

current directory.   Use this  option  if you  believe the  program  is

misbehaving,  and  show  the  resulting   log  to  your  local   Kermit

maintainer. (Debug logging is disabled.)

-h      Help -- Display a brief synopsis of the command line options.

-u d    File handle -- The  argument d is the numerical  value of an open  file

handle which Kermit will use for  its communication line.  This  option

is for use only by other programs which interface with Kermit, not  for

typing in  the  command  line  (so  it's  not  described  in  the  help

information from -h). Further details are given in the Appendix.

The command line may contain no more  than one protocol action option (ie  only

one of: -s, -a, -r, -k, -x, -g, -f).

Files are sent with their own  names, except that lowercase letters are  raised

to upper, drive specifiers and pathnames are stripped off, and non-alphanumeric

characters (excepting the dot)  are changed to 'X'.  Incoming files are  stored

under their own names.  If -w was specified, a "generation number" is  appended

to the name if it has the same  name as an existing file which would  otherwise

be overwritten.  If the -a option is included, then the same rules apply to its

argument.  The file transfer display  shows any transformations performed  upon

filenames.

During transmission, files are encoded as follows:

- Control characters are converted to prefixed printables.

- Sequences of repeated characters are  collapsed via repeat counts, if  the

other Kermit is also capable of repeated-character compression.

- If parity is being  used on the communication  line, data characters  with

the 8th (parity) bit on are specially prefixed (provided the other  Kermit

is capable of  8th-bit prefixing;  if not,  8-bit binary  files cannot  be

successfully transferred).

4.2 Command Line Examples

kermit -l com1 -b 1200 -cn -r

This command connects you to the system on the other end of COM1 at 1200  baud,

where you presumably log in  and run Kermit with  a 'send' command.  After  you

escape back, C-Kermit waits  for a file  (or files) to  arrive.  When the  file

transfer is completed, you are reconnected to the remote system so that you can

logout.

kermit -l com2 -b 9600 -cntp m -r -a foo.bar

This command is like the preceding one,  except the remote system in this  case

uses half duplex communication with mark  parity.  The first file that  arrives

is stored under the name foo.bar.

kermit -nf

This command would be used to shut down a remote server and then connect to the

remote system, in order to log out or to make further use of it.  The -n option

is invoked after -f (-c would have been invoked before).  This example  assumes

that the remote server is connected to COM1 (Kermit's default comms port),  and

that the current baud rate of the port is acceptable to the remote server.

kermit -wx

This command starts up C-Kermit as a server.  Incoming files that have the same

names as existing files are given new, unique names.

kermit -l com2 -b 9600

This command  sets  the communication  line  and  speed.  Since  no  action  is

specified, C-Kermit issues a prompt and enters an interactive dialog with  you.

Any settings  given on  the command  line remain  in force  during the  dialog,

unless explicitly  changed  (eg in  this  case by  'set  line' or  'set  speed'

commands).

kermit

This command  starts up  Kermit interactively  with all  default settings.  The

serial line used will be COM1, and the speed used will be the current speed  of

COM1.

A final example shows  how a (hypothetical) OS/2  compression utility might  be

used to speed up Kermit file transfers:

compress < file | kermit -is -     (sender)

kermit -ik | uncompress > file     (receiver)

4.3 Exit Status Codes

C-Kermit returns  an  exit  status  of  zero, except  when  a  fatal  error  is

encountered, when the exit status is set to  one.  This can be used in a  batch

file, to take some  action depending on whether  the operation was  successful.

For instance, suppose the file SEND.CMD contains the following:

echo Sending %1 out port %2

kermit -ql COM%2 -b 9600 -s %1

if ERRORLEVEL 1 goto badend

echo Transferred succcessfully!

goto end

:badend

echo Transfer problems!

:end

To send a file FOO.BAS, you could type:

send foo.bas 2

to send it to another computer running Kermit, connected to port COM2.  If  the

transfer completed OK, you would get the message 'Transferred successfully!'.

5 INTERACTIVE OPERATION

C-Kermit's interactive  command prompt  is "C-Kermit>".   In response  to  this

prompt, you may type any valid interactive C-Kermit command.  C-Kermit executes

the command and then  prompts you for another  command.  The process  continues

until you instruct the program to terminate.

Commands begin with a keyword, normally  an English verb, such as 'send'.   You

may omit  trailing  characters  from  any  keyword,  so  long  as  you  specify

sufficient characters to distinguish  it from any other  keyword valid in  that

field.  Certain commonly-used keywords  (such as 'send', 'receive',  'connect')

also have special non-unique abbreviations ('s' for 'send', 'r' for  'receive',

'c' for 'connect').

Certain  characters  have  special  functions  during  typein  of   interactive

commands:

?   Question mark, typed at any point in a command, will produce a  message

explaining what is possible  or expected at  that point.  Depending  on

the context, the message may be a brief phrase, a menu of keywords,  or

a list of files.

ESC (The Escape  key)  -- Request  completion  of the  current  keyword  or

filename, or insertion of a default  value.  The result will be a  beep

if the requested operation fails.

BS  (Backspace) -- Delete the previous character from the command.

^W  (Control-W) -- Erase the rightmost word from the command line.

^U  (Control-U) -- Erase the entire command.

^R  (Control-R) -- Redisplay the current command.

SP  (Space) --  Delimits fields  (keywords,  filenames, numbers)  within  a

command.  The tab key may also be used for this purpose.

CR  (Carriage Return or  Enter) --  Enters the command  for execution.   LF

(Linefeed or Control-J) or FF (formfeed or Control-L) may also be  used

for this purpose.

^   (Circumflex) -- Enter  any of  the above characters  into the  command,

literally.  To enter a ^, type two of them in a row (^^).  A circumflex

at the end of a command  line causes the next line  to be treated as  a

continuation line; this  is useful  for readability  in command  files,

especially in the 'script' command.

You may type the editing characters (BS, ^W, etc) repeatedly, to delete all the

way back to  the prompt.   No action  will be  performed until  the command  is

entered by typing  carriage return,  linefeed, or  formfeed.  If  you make  any

mistakes, you will  receive an informative  error message and  a new prompt  --

make liberal use of '?' and 'ESC'  to feel your way through the commands.   One

important command  is 'help'  --  you should  use it  the  first time  you  run

C-Kermit.

A command line beginning with a percent sign '%' is ignored.  Such lines may be

used to include illustrative commentary in Kermit command dialogs.

Interactive C-Kermit accepts commands from files as well as from the  keyboard.

When you  start C-Kermit,  the program  looks  for a  file CKERMIT.INI  in  the

current PATH,  and executes  any  commands it  finds  there.  The  commands  in

CKERMIT.INI must be in interactive format,  not in the command-line format.   A

'take' command  is also  provided for  use at  any time  during an  interactive

session, to  allow interactive-format  commands to  be  executed from  a  file;

command files may be nested to any reasonable depth.

Here is a brief list of C-Kermit interactive commands:

%  Comment

!  Execute an OS/2 command, or start up CMD.EXE.

bye  Terminate and log out a remote Kermit server.

close  Close a log file.

connect  Establish a terminal connection to a remote system.

cwd  Change Working Directory.

dial  Dial a telephone number.

directory  Display a directory listing.

echo  Display arguments literally.

exit  Exit from the program, closing any open files.

finish  Tell remote Kermit server to exit, but not log out.

get  Get files from a remote Kermit server.

help  Display a help message for a given command.

log  Open log file: debugging, packet, session, transaction.

quit  Same as 'exit'.

receive  Passively wait for files to arrive.

remote  Do some file management on a remote Kermit server.

script  Execute a login script with a remote system.

send  Send files.

server  Begin server operation.

set  Set various parameters.

show  Display values of 'set' parameters.

space  Display current disk space usage.

statistics  Display statistics about most recent transaction.

take  Execute commands from a file.

The 'set' parameters are:

block-check  Level of packet error detection.

delay  How long to wait before sending first packet.

duplex  Specify which side echoes during 'connect'.

escape-character  Prefix for "escape commands" during 'connect'.

file  Set various file parameters.

flow-control  Communication line full-duplex flow control.

handshake  Communication line half-duplex turnaround character.

incomplete  Disposition for incompletely received files.

line  Communication line device name.

modem-dialer  Type of modem-dialer on communication line.

parity  Communication line character parity.

prompt  The C-Kermit program's interactive command prompt.

receive  Parameters for inbound packets.

retry  Packet retransmission limit.

send  Parameters for outbound packets.

speed  Communication line speed.

terminal  Terminal parameters.

The 'remote' commands are:

cwd  Change remote working directory.

delete  Delete remote files.

directory  Display a listing of remote file names.

help  Request help from a remote server.

host  A command to the remote host in its command language.

space  Display current disk space usage on remote system.

type  Display a remote file on your screen.

who  Show who's logged in, or get information about a user.

Most of these commands are described adequately in the Kermit User Guide or the

Kermit book.  Special aspects of certain C-Kermit commands are described below.

5.1 The 'send' command

Syntax:     send fn

or          send fn1 rfn1

Send the file  or files  denoted by  fn to the  other Kermit,  which should  be

running as a  server, or which  should have been  given the 'receive'  command.

Each file is sent under  its own name (as described  above, or as specified  by

the 'set file names'  command).  If the  second form of  the 'send' command  is

used, i.e.  with fn1 denoting  a single OS/2 file, rfn1  may be specified as  a

name to send  it under.  The  'send' command  may be abbreviated  to 's',  even

though 's' is not a unique abbreviation for a top-level C-Kermit command.

The wildcard  characters '*'  and '?'  are accepted  in fn.   If '?'  is to  be

included, it  must  be prefixed  by  '^' to  override  its normal  function  of

providing help.   '*' matches  any string,  '?' matches  any single  character.

When fn contains '*' or '?' characters, there is a limit to the number of files

that can be matched,  which varies depending  on the length  of the file  names

involved.  If you get the  message 'Too many files  match' then you'll have  to

make a more judicious selection.

5.2 The 'receive' command

Syntax:     receive

or          receive fn1

Passively wait for files to arrive from  the other Kermit, which must be  given

the 'send' command -- the 'receive' command does not work in conjunction with a

server (use 'get'  for that).  If  fn1 is specified,  store the first  incoming

file under that name.  The 'receive' command may be abbreviated to 'r'.  If the

second form of  the command is  given, and fn1  contains a path  specification,

then that path must exist -- C-Kermit will not create a directory for the file.

5.3 The 'get' command

Syntax:     get rfn

or          get

rfn

fn1

Request a remote Kermit server to send the named file or files.  Since a remote

file specification  (or  list) might  contain  spaces, which  normally  delimit

fields of a C-Kermit command, an alternate  form of the command is provided  to

allow the inbound file to be given a new name: type 'get' alone on a line,  and

you will be prompted separately for  the remote and local file  specifications,

for example

C-Kermit>get

Remote file specification: profile exec

Local name to store it under: profile.ibm

As with 'receive',  if more  than one  file arrives as  a result  of the  'get'

command, only the first will be stored  under the alternate name given by  fn1;

the remaining files will be stored under their own names if possible.  If a '?'

is to be included in the remote file specification, you must prefix it with '^'

to suppress its normal function of providing help.

If you have started a multiline 'get'  command, you may escape from its  lower-

level prompts by typing a carriage return in response to the prompt, e.g.

C-Kermit>get

Remote file specification: foo

Local name to store it under: (Type a carriage return here)

(cancelled)

C-Kermit>

5.4 The 'server' command

The 'server' command places C-Kermit in "server mode" on the currently selected

communication line.  All further commands must  arrive as valid Kermit  packets

from the Kermit on  the other end  of the line.  The  OS/2 C-Kermit server  can

respond to the following commands:

Command               Server Response

get                   Sends files

send                  Receives files

bye                   Attempts to log itself out

finish                Exits to level from which it was invoked

remote directory      Sends directory lising

remote delete         Removes files

remote cwd            Changes working directory

remote type           Sends files to your screen

remote space          Reports about its disk usage

remote help           Lists these capabilities

remote host           Execute an OS/2 command

Note that the 'remote host' command should be used with great care.  It  should

only be used to invoke OS/2 commands which produce their output through  stdio,

and which  require  no  keyboard interaction.   Commands  such  as  'copy'  and

'rename' are OK (although  they may sometimes produce  output on stderr,  which

will appear on the screen of the OS/2  system).  It is not possible to use  the

'remote host' command to run a word proccessor, for instance.

5.5 The 'remote', 'bye', and 'finish' commands

C-Kermit may itself request services from a remote Kermit server.  In  addition

to 'send' and 'get', the following commands may also be sent from C-Kermit to a

Kermit server:

remote cwd [directory ]

If the optional remote directory specification is included, you will be

prompted on a separate line for a password, which will not echo as  you

type it.  If  the remote system  does not require  a password for  this

operation, just type a carriage return.

remote delete rfn

delete remote file or files.

remote directory [rfn ]

directory listing of remote files.

remote host command

issue a command in the remote host's own command language.

remote space

disk usage report from the remote host.

remote type rfn

display remote file or files on the screen.

remote who [user ]

display information about who's logged in.

remote help

display remote server's capabilities.

bye and finish

When connected  to a  remote Kermit  server, these  commands cause  the

remote server to  terminate; 'finish'  returns it to  Kermit or  system

command level (depending on the implementation  or how the program  was

invoked); 'bye' also requests it to log itself out.

5.6 The 'log' and 'close' commands

Syntax:     log {debugging, packets, session, transactions} [fn1 ]

close {debugging, packets, session, transactions}

C-Kermit's progress may be logged in  various ways.  The 'log' command opens  a

log, the 'close' command closes it.  In  addition, all open logs are closed  by

the 'exit' and 'quit' commands.  A name may be specified for a log file; if the

name is omitted, the file is created with a default name as shown below.

log debugging

This produces a voluminous log of the internal workings of C-Kermit, of

use to Kermit developers or maintainers in tracking down suspected bugs

in the C-Kermit program.  Use  of this feature dramatically slows  down

the Kermit  protocol.   Default  name:  debug.log.  (Debug  logging  is

disabled.)

log packets

This produces a record  of all the  packets that go in  and out of  the

communication port.  This log is of  use to Kermit maintainers who  are

tracking down protocol problems in  either C-Kermit or any Kermit  that

C-Kermit is connected to.  Default name: packet.log.

log session

This log  will contain  a copy  of everything  you see  on your  screen

during the 'connect' command, except for local messages or  interaction

with local escape commands.  Default name: session.log.

log transactions

The transaction log  is a record  of all  the files that  were sent  or

received while transaction  logging was  in effect.   It includes  time

stamps and  statistics, filename  transformations, and  records of  any

errors that may have occurred.  The transaction log allows you to  have

long unattended file  transfer sessions  without fear  of missing  some

vital screen message.  Default name: transact.log.

The 'close' command explicitly closes a log, e.g. 'close debug'.

Note: Debug and  Transaction logs are  a compile-time option;  C-Kermit may  be

compiled without these logs, in which case it will run faster, it will take  up

less space on the disk, and the commands relating to them will not be present.

5.7 Local File Management Commands

OS/2 Kermit  allows  some degree  of  local file  management  from  interactive

command level:

directory [fn ]

Displays a listing of the names, sizes, and dates of files matching  fn

(which defaults to '*.*').  Equivalent to 'dir'.

cwd directory-name

Changes Kermit's working  directory to  the one  given.  The  directory

specification may be preceeded by a drive specifier, in which case that

becomes the  current  drive.   This command  affects  only  the  Kermit

process and any processes it may subsequently create.

space

Display information  about  disk  space and/or  quota  in  the  current

directory and device.  Equivalent to 'chkdsk'.

! [command ]

The command is executed by the OS/2 command interpreter CMD.EXE.  If no

command is specified, then CMD.EXE itself is started; terminating it by

typing 'exit' will return you to  C-Kermit command level.  Use the  '!'

command to provide  file management or  other functions not  explicitly

provided  by   C-Kermit  commands.    The  '!'   command  has   certain

peculiarities:

- At least one space must separate the '!' from the command.

- A 'cd'  or 'chdir'  (change directory)  command executed  in  this

manner will  have no  effect on  returning to  Kermit --  use  the

C-Kermit 'cwd' command instead.

5.8 The 'set' Command

Syntax:     set parameter [parameter] value

Since Kermit is designed to allow  diverse systems to communicate, it is  often

necessary to  issue special  instructions  to allow  the  program to  adapt  to

peculiarities  of  the  another  system  or  the  communication  path.    These

instructions are accomplished by the 'set' command.  The 'show' command may  be

used to  display  current settings.   Here  is  a brief  synopsis  of  settings

available in the current release of C-Kermit:

block-check {1, 2, 3}

Determines the level of per-packet error  detection.  "1" is a  single-

character 6-bit checksum, folded to include the values of all bits from

each character.   "2" is  a  2-character, 12-bit  checksum.  "3"  is  a

3-character, 16-bit  cyclic redundancy  check  (CRC).  The  higher  the

block check,  the better  the error  detection and  correction and  the

higher the resulting  overhead.  Type 1  is most commonly  used; it  is

supported by all Kermit implementations, and it has proven adequate  in

most circumstances.   Types 2  or 3  would be  used to  advantage  when

transferring 8-bit binary files over noisy lines.

delay n

How many seconds to wait before sending the first packet after a 'send'

command, in remote mode only.  It is irrelevant for OS/2 Kermit,  since

it is always in local mode.

duplex {full, half}

For use during 'connect'.  Specifies which  side is doing the  echoing;

'full' means the  other side,  'half' means C-Kermit  must echo  typein

itself.

escape-character cc

For use  during  'connect' to  get  C-Kermit's attention.   The  escape

character acts as  a prefix  to an  'escape command',  for instance  to

close the connection and return to C-Kermit or OS/2 command level.  The

normal escape character is Control-] (29).

file {display, names, type, warning}

Establish various file-related parameters:

display {on, off}

Normally 'on'; when  in local  mode, display  progress of  file

transfers on the  screen (stdout), and  listen to the  keyboard

for interruptions.   If 'off'  (equivalent to  '-q' on  command

line) none of this is done,  and the file transfer may  proceed

in the background oblivious to any other work concurrently done

at the console terminal.

names {converted, literal}

Normally 'converted, which means  that outbound filenames  have

path specifications  stripped and  non-alphanumeric  characters

changed to X's (except for the dot).  'literal' means that none

of these conversions  are done; therefore,  any directory  path

appearing in a  received file specification  must exist and  be

write-accessible.  When  literal  naming  is  being  used,  the

sender should  not use  path names  in the  file  specification

unless the  same  path  exists on  the  target  system  and  is

writable.

type {binary, text} [{7, 8}]

The file type is normally text, which means that any  control-Z

in a file being transmitted is treated as an end-of-file  mark.

Binary means transmit file contents without conversion.  Binary

('-i' in command line notation) is necessary for binary files.

The optional  trailing parameter  tells the  bytesize for  file

transfer. It is 8 by default.  If you specify 7, the high order

bit will be stripped from each byte of sent and received files.

This is  useful  for  transferring text  files  that  may  have

extraneous high  order bits  set in  their disk  representation

(e.g.  Wordstar or similar word processor files).

warning {on, off}

Normally 'off', which means  that incoming files will  silently

overwrite existing files of the same name.  When 'on' ('-w'  on

command line)  Kermit  will check  if  an arriving  file  would

overwrite an existing file; if so, it will construct a new name

for the arriving file, of  the form FZZn.BAR, where FZZ.BAR  is

the name they share and n is a "generation number"; if  FZZ.BAR

exists, then  the new  file will  be called  FZZ00001.BAR.   If

FZZ.BAR  and  FZZ00001.BAR   exist,  the  new   file  will   be

FZZ00002.BAR, and so on.  If the  common name were more than  6

characters long (eg  GOODDATA.DAT), then the  new name for  the

arriving file would be GOODD001.DAT and so on.

CAUTION: If Control-F or Control-B is used to cancel an

incoming file, and a file  of the same name  previously

existed, and the "file warning" feature is not enabled,

then the previous copy of the file will disappear.

flow-control {none, xon/xoff}

Normally 'xon/xoff' for  full duplex flow  control.  Should  be set  to

'none' if the other system cannot  do xon/xoff flow control, or if  you

have issued  a 'set  handshake' command.   If set  to 'xon/xoff',  then

'handshake' should be set to 'none'.  This setting applies during  both

terminal connection and file transfer.

incomplete {discard, keep}

Disposition for incompletely  received files.  If  an incoming file  is

interrupted or  an error  occurs  during transfer,  the part  that  was

received so far is  normally discarded.  If  you 'set incomplete  keep'

then such file fragments will be kept.

handshake {xon, xoff, cr, lf, bell, esc, none}

Normally 'none'.  Otherwise, half-duplex communication line  turnaround

handshaking is done,  which means  Kermit will  not reply  to a  packet

until it has received  the indicated handshake  character or has  timed

out waiting  for it;  the handshake  setting applies  only during  file

transfer.  If you  'set handshake'  to other than  'none', then  'flow'

should be set to 'none'.

line [dev ]

The device name for the communication line to be used for file transfer

and terminal  connection, e.g.  COM2.  If  you  omit the  device  name,

Kermit will revert to its default device, COM1.

modem-dialer {direct, hayes, racalvadic, ventel, ...}

The type of modem dialer on the communication line.  'direct' indicates

either there is no dialout modem, or that if the line requires  carrier

detection to open, then  'set line' will hang  waiting for an  incoming

call.  'hayes', 'ventel', and the  others indicate that 'set line'  (or

the '-l' argument) will prepare for a subsequent 'dial' command for the

given dialer.  Support for new dialers  is added from time to time,  so

type 'set  modem ?'  for a  list of  those supported  in your  copy  of

Kermit. See the description of the 'dial' command.

parity {even, odd, mark, space, none}

Specify character parity  for use in  packets and terminal  connection,

normally 'none'.  If other than 'none',  C-Kermit will seek to use  the

8th-bit prefixing mechanism for  transferring 8-bit binary data,  which

can be used successfully only if the other Kermit agrees; if not, 8-bit

binary data cannot be successfully transferred.

prompt [string ]

The given string will be substituted for 'C-Kermit>' as this  program's

prompt.   If  the  string  is  omitted,  the  prompt  will  revert   to

'C-Kermit>'.  If the string  is enclosed in  double quotes, the  quotes

will be stripped and any leading and trailing blanks will be retained.

send parameter

Establish parameters to  use when sending  packets.  These  will be  in

effect only for  the initial packet  sent, since the  other Kermit  may

override  these  parameters  during  the  protocol  parameter  exchange

(unless noted below).

end-of-packet cc

Specifies the control character needed  by the other Kermit  to

recognize the end of a  packet.  C-Kermit sends this  character

at the  end of  each packet.   Normally 13  (carriage  return),

which  most  Kermit  implementations  require.   Other  Kermits

require no  terminator  at  all, still  others  may  require  a

different terminator, like linefeed (10).

packet-length n

Specify the  maximum  packet  length  to  send.   Normally  90.

Shorter packet lengths can  be useful on  noisy lines, or  with

systems or front ends or networks that have small buffers.  The

shorter the packet, the higher the overhead, but the lower  the

chance of a packet being corrupted by noise, and the less  time

to retransmit corrupted  packets.  This  command overrides  the

value requested by the other Kermit during protocol  initiation

unless the other Kermit requests a shorter length.

pad-character cc

Designate a character  to send before  each packet.   Normally,

none is  sent.  Outbound  padding  is sometimes  necessary  for

communicating with  slow half  duplex systems  that provide  no

other means of line turnaround control.  It can also be used to

send special characters to communications equipment that  needs

to be put in "transparent" or "no echo" mode, when this can  be

accomplished in by feeding it a certain control character.

padding n

How many pad characters to send, normally 0.

start-of-packet cc

The normal Kermit packet prefix is Control-A (1); this  command

changes the prefix C-Kermit puts on outbound packets.  The only

reasons this should  ever be changed  would be:  Some piece  of

equipment somewhere between  the two Kermit  programs will  not

pass through  a  Control-A;  or, some  piece  of  of  equipment

similarly placed is echoing its input.  In the latter case, the

recipient of  such an  echo can  change the  packet prefix  for

outbound packets to be different from that of arriving packets,

so that  the  echoed packets  will  be ignored.   The  opposite

Kermit must also be told to  change the prefix for its  inbound

packets.

timeout n

Specifies the number of  seconds you want  the other Kermit  to

wait  for  a  packet  before  timing  it  out  and   requesting

retransmission.  Defaults to 10 seconds.

receive parameter

Establish parameters to request  the other Kermit  to use when  sending

packets.

end-of-packet cc

Requests the other  Kermit to  terminate its  packets with  the

specified character.

packet-length n

Specify the maximum packet  length to that  you want the  other

Kermit to send, normally 90.  If you specify a length of 95  or

greater, then it will be used if the other Kermit supports, and

agrees to use, the Kermit protocol extension for long  packets.

In this  case,  the maximum  length  depends upon  the  systems

involved, but there would normally be no reason for packets  to

be more  than  about  1000 characters  in  length.   The  'show

parameters' command  displays  C-Kermit's current  and  maximum

packet lengths.

pad-character cc

C-Kermit normally  does  not  need  to  have  incoming  packets

preceded with pad characters.  This command allows C-Kermit  to

request the other Kermit to use cc as a pad character.  Default

cc is NUL, ASCII 0.

padding n

How many pad characters to ask for, normally 0.

start-of-packet cc

Change the  prefix C-Kermit  looks for  on inbound  packets  to

correspond with what the other Kermit is sending.

timeout n

Normally, each Kermit partner sets its packet timeout  interval

based on  what  the  opposite Kermit  requests.   This  command

allows you  to  override the  normal  procedure and  specify  a

timeout interval  for  OS/2  Kermit to  use  when  waiting  for

packets from  the other  Kermit.  If  you  specify 0,  then  no

timeouts will  occur, and  OS/2 Kermit  will wait  forever  for

expected packets to arrive.

speed {0, 110, 150, 300, 600, 1200, 2400, 4800, 9600, 19200}

The baud rate  for the external  communication line.  'set  baud' is  a

synomym for 'set speed'.  19200 baud may not be available, depending on

your communications hardware.

terminal parameter value

Used for specifying terminal parameters.  Currently, 'bytesize' is  the

only parameter provided, and it  can be set to  7 or 8.  This  controls

the width of the  data path between the  console and the remote  system

when C-Kermit is in  connect (ie terminal emulation)  mode.  It's 7  by

default.

5.9 The 'show' Command

Syntax:     show {parameters, versions}

The 'show'  command with  the  default argument  of 'parameters'  displays  the

values of  all  the  'set'  parameters described  above.   If  you  type  'show

versions', then C-Kermit will display the version numbers and dates of all  its

internal modules.  You should use the 'show versions' command to ascertain  the

vintage of your Kermit program before reporting problems to Kermit maintainers.

5.10 The 'statistics' Command

The statistics  command  displays  information about  the  most  recent  Kermit

protocol transaction, including  file and  communication line  i/o, timing  and

efficiency, as well as  what encoding options were  in effect (such as  8th-bit

prefixing, repeat-count compression).

5.11 The 'take' and 'echo' Commands

Syntax:     take fn1

echo [text to be echoed]

The 'take' command instructs C-Kermit to execute commands from the named  file.

The file  may  contain any  interactive  C-Kermit commands,  including  'take';

command files may be  nested to any  reasonable depth, but  it may not  contain

text to be sent to  a remote system during  the 'connect' command.  This  means

that a command file like this:

set speed 9600

connect

login myuserid

mypassword

etc

will not  send "login  myserid" or  any of  the following  text to  the  remote

system.  To  carry on  a canned  dialog, use  the 'script'  command,  described

later.

The '%' command is useful for including comments in take-command files.  It may

only be used at the beginning of a line.

The 'echo'  command  may be  used  within  command files  to  issue  greetings,

announce progress, ring the terminal  bell, etc.  This command simply  displays

its text argument (almost) literally at the terminal; the argument may  contain

octal escapes of the form \ooo, where o is an octal digit (0-7), and there  may

be 1, 2, or 3 such digits, whose value specify an ASCII character, such as \007

(or \07 or just \7) for beep, \012 for newline, etc.

Take-command files  are in  exactly the  same syntax  as interactive  commands.

Note that this  implies that  if you want  to include  special characters  like

question mark or circumflexes that you would  have to quote with ^ when  typing

interactive commands, you must quote these  characters the same way in  command

files.  Long lines may be continued by ending them with a single ^.

Command files  may be  used in  lieu of  command macros,  which have  not  been

implemented in this version of C-Kermit.  For instance, if you commonly connect

to a system called 'B' that is connected to com2 at 4800 baud, you could create

a file called b containing the commands

% C-Kermit command file to connect to System B thru com2

set line com2

set speed 4800

% Beep and give message

echo \007Connecting to System B...

connect

and then simply type 'take b' (or 't b' since no other commands begin with  the

letter 't') whenever you  wish to connect to  system B. Note the comment  lines

and the beep inserted into the 'echo' command.

For connecting to  IBM mainframes,  a number  of 'set'  commands are  required;

these, too, can be conveniently collected into a 'take' file like this one:

% Sample C-Kermit command file to set up current line

% for IBM mainframe communication

%

set parity mark

set handshake xon

set flow-control none

set duplex half

Note that no single command is available to wipe out all of these settings  and

return C-Kermit  to its  default startup  state;  to do  that, you  can  either

restart the program, or  else make a command  file that executes the  necessary

'set' commands:

% Sample C-Kermit command file to restore normal settings

%

set parity none

set handshake none

set flow-control xon/xoff

set duplex full

An implicit 'take' command is executed upon your CKERMIT.INI file when C-Kermit

starts up, upon either interactive or command-line invocation.  The CKERMIT.INI

file should contain 'set'  or other commands  you want to be  in effect at  all

times.  For instance, you might want override the default action when  incoming

files have the same names as existing files -- in that case, put the command

set file warning on

in your CKERMIT.INI file.

Errors encountered during execution of take files (such as failure to  complete

dial or script operations) cause termination of the current take file,  popping

to the level  that invoked  it (take file,  interactive level,  or the  command

interpreter).

You may  also  use the  redirection  mechanism  to cause  C-Kermit  to  execute

commands from a file:

kermit < cmdfile

or you can even pipe commands in from another process:

genkcmds | kermit

5.12 The 'connect' Command

The 'connect' command ('c' is an acceptable non-unique abbreviation) links your

terminal to another computer as if it  were a local terminal to that  computer,

through the  device specified  in  the most  recent  'set line'  command.   All

characters you type at your keyboard  are sent out the communication line  (and

if you have 'set duplex half',  also displayed on your screen), and  characters

arriving at  the  communication port  are  displayed on  the  screen.   Current

settings of speed, parity, duplex, and  flow-control are honored, and the  data

connection is  7 bits  wide unless  you have  given the  command 'set  terminal

bytesize 8'.  If you have issued a 'log session' command, everything you see on

your screen will also be recorded to your session log.  This provides a way  to

"capture" files from remote systems that don't have Kermit programs available.

To get back to your  own system, you must type  the escape character, which  is

Control-] (^]) (unless  you have  changed it  with the  'set escape'  command),

followed by a  single-character command,  such as 'c'  for "close  connection".

Single-character commands include:

c     Close the connection

b     Send a BREAK signal for about 1 second

0     (zero) send a null

?     Display help information about these options

h     Hangup the phone

^]    Send Control-] itself (whatever you  have defined the escape  character

to be, typed twice in a row sends one copy of it).

Uppercase and  control  equivalents  for  (most  of)  these  letters  are  also

accepted.  A space  typed after  the escape  character is  ignored.  Any  other

character will produce a beep.

In connect  mode, C-Kermit  emulates a  DEC VT102  terminal.  See  the  section

"Terminal Emulation" for details of how the emulation works.

Note that when in interactive command  mode, C-Kermit reads its keyboard  input

from stdin  and  writes its  screen  output to  stdout,  allowing  command-line

redirection to  be used  as described  in the  previous section.   However,  in

connect mode, keyboard input is obtained through the KBD subsystem, and  screen

output is through the  VIO subsystem.  It is  therefore impossible to  redirect

terminal I/O.

5.13 The 'dial' command

Syntax:     dial telephone-number-string

This command controls  dialout modems; you  should have already  issued a  'set

line' and  'set speed'  command to  identify the  terminal device,  and a  'set

modem' command to identify the  type of modem to be  used for dialing.  In  the

'dial' command, you supply the phone number and the Kermit program feeds it  to

the modem in the appropriate format and then interprets dialer return codes and

modem signals to  inform you whether  the call was  completed.  The  telephone-

number-string may contain  imbedded modem-dialer  commands, such  as comma  for

Hayes pause, or '&' for Ventel dialtone wait and '%' for Ventel pause  (consult

your modem manual for details).

At the time of this writing, support is included for the following modems:

- AT&T 7300 Internal Modem

- Cermetek Info-Mate 212A

- Concord Condor CDS 220

- DEC DF03-AC

- DEC DF100 Series

- DEC DF200 Series

- General DataComm 212A/ED

- Hayes Smartmodem 1200 and compatibles

- Penril

- Racal Vadic

- US Robotics 212A

- Ventel

A number of these modems  are not generally found  connected to PCs.  The  most

common modem type used with an OS/2 system is "Hayes compatible".  Support  for

new modems is added to the program from time to time; you can check the current

list by typing 'set modem ?'.

The device used for  dialing out is the  one selected in  the most recent  'set

line' command.

Example:

kermit -l com1 -b 1200

C-Kermit>set modem-dialer hayes         hint: abbreviate set m h

C-Kermit>dial 9,5551212

Connected!

C-Kermit>connect                        hint: abbreviate c

logon, request remote server, etc.

^]c                                     escape back

C-Kermit> ...

C-Kermit>quit                           hint: abbreviate q

this disconnects modem, and hangs up the line.

C-Kermit requires that  the modem  track the computer's  "data terminal  ready"

(DTR) signal.  If a switch setting is available to simulate DTR asserted within

the modem, then it should normally not be in that setting.  Otherwise the modem

will be unable to hang up at the end of a call.

For Hayes dialers, two important switch settings  are no. 1 and no. 6.   Switch

no. 1 should  be normally be  UP so that  the modem can  act according to  your

computer's DTR signal.  Switch no. 6  should normally be  UP so  carrier-detect

functions properly (but put it DOWN if you have trouble with the UP  position).

Switches no. 2  (English versus  digit result codes)  and no.  4 (Hayes  echoes

modem commands) may be in either position.

5.14 The 'script' Command

Syntax:     script expect send [expect send] . . .

"expect" has the syntax: expect[-send-expect[-send-expect[...]]]

The 'script' command  carries on  a "canned dialog"  with a  remote system,  in

which data is sent according to the remote system's responses.  The typical use

is for logging in to a remote system automatically.

C-Kermit's script facility operates in a  manner similar to that commonly  used

by the Unix UUCP system's 'L.sys' file  entries.  A login script is a  sequence

of the form:

expect send [expect send] . . .

where expect is a prompt or message to  be issued by the remote site, and  send

is the string (names, numbers, etc)  to return, and expects are separated  from

sends by spaces.  The send may also be  the keyword EOT, to send Control-D,  or

BREAK, to send a break signal.  Letters in sends may be prefixed by '~' to send

special characters, including:

~b        backspace

~s        space

~q        '?'(trapped by Kermit's command interpreter)

~n        linefeed

~r        carriage return

~t        tab

~'        single quote

~~        tilde

~"        double quote

~x        XON (Control-Q)

~c        don't append a carriage return

~d        delay approx 1/3 second during send

~o[o[o]]  an octal character

~w[d[d]]  wait specified interval during expect, then time out

As with some UUCP systems, sent strings  are followed by ~r unless they have  a

~c.

Only the last 7 characters in each expect are matched.  A null expect, e.g.  ~0

or two adjacent dashes, causes a short delay before proceeding to the next send

sequence.  A null expect always succeeds.

As with UUCP, if the expect string  does not arrive, the script attempt  fails.

If you  expect that  a sequence  might not  arrive, as  with UUCP,  conditional

sequences may be expressed in the form:

-send-expect[-send-expect[...]]

where dashed sequences are followed as long as previous expects fail.  Timeouts

for expects can be specified using ~w; ~w with no arguments waits 15 seconds.

Expect/send transactions can  be easily  be debugged  by logging  transactions.

This records all  exchanges, both expected  and actual.   The script  execution

will also be logged in the session log, if that is activated.

Note that '^' characters in login scripts, as in any other C-Kermit interactive

commands, must  be doubled  up.  A  line may  be ended  with a  single '^'  for

continuation.

Example:

Using a Hayes-compatible  modem, dial  up a  PAD, simulating  pressing CR  four

times to get the 'PAD>' prompt.  (Note how '~0' stands for a null expect string

and for a null send string - ie just send a carriage return.) Call a VAX system

named 'ERCVAX'.  Get  the ..name: prompt,  and respond with  the user name  and

password.  Notice that the '^' character is used to continue the script command

onto the next line.

set modem hayes

set line com2

set baud 1200

dial 0319871234

script ~0 ~0 ~0 ~0 ~0 ~0 ~0 ~0 PAD> CALL~sERCVAX ^

name: SMITH word: SECRET

Note that 'set  line' is issued  after 'set  modem', but before  'set baud'  or

other line-related parameters.

5.15 The 'help' Command

Syntax:     help

or          help keyword

or          help {set, remote} keyword

Brief help messages or menus are always available at interactive command  level

by typing a question mark at any point.   A slightly more verbose form of  help

is available through the 'help' command.  The 'help' command with no  arguments

prints a brief summary of  how to enter commands and  how to get further  help.

'help' may be followed by one of the top-level C-Kermit command keywords,  such

as 'send', to request information about a command.  Commands such as 'set'  and

'remote' have a further level of help.   Thus you may type 'help', 'help  set',

or 'help set parity'; each will  provide a successively more detailed level  of

help.

5.16 The 'exit' and 'quit' Commands

These two commands are identical.  Both of them do the following:

- Relinquish access to any communication line assigned via 'set line'.

- Hang up the modem, if any, by dropping DTR.

- Close any open logs or other files.

- Exit the program.

After exit from C-Kermit, your current directory  will be the same as when  you

started the program.  The 'exit' command is issued implicitly whenever C-Kermit

halts normally, e.g. after a command line invocation.

6 TERMINAL EMULATION

When you issue a  'connect' command the first  time after starting Kermit,  the

screen clears and the  cursor is positioned at  the top left-hand corner.   You

can log into the remote host computer as normal.  In this mode, the PC emulates

a DEC VT102 terminal,  so any control codes  or escape sequences received  from

the host will be actioned appropriately.

The 25th line on the screen  is used as a status  line, giving the name of  the

comms port, the current baud rate and how to obtain help.

Some keys on the VT102 keyboard have  no direct equivalent on the PC  keyboard.

The following table shows the mapping  which obtains between VT102 keys and  PC

keys.  Note that the Alt n combinations use  the number keys along the top  row

of the keyboard, not the numeric keypad.

VT102               IBMPC

Delete              Del

PF1                 F1

PF2                 F2

PF3                 F3

PF4                 F4

Keypad 0            Alt 0

Keypad 1            Alt 1

Keypad 2            Alt 2

Keypad 3            Alt 3

Keypad 4            Alt 4

Keypad 5            Alt 5

Keypad 6            Alt 6

Keypad 7            Alt 7

Keypad 8            Alt 8

Keypad 9            Alt 9

Keypad minus        F5 or F6

Keypad comma        F7 or F8

Keypad dot          F9

Keypad enter        F10

No Scroll           Home

The PC's 'Home' key (equivalent to the VT102 'No Scroll' key) freezes the  data

on the screen.   It is  typically used  when listing  a long  file, to  prevent

information being scrolled off the top of the screen.  Note that the  Control-S

and Control-Q (Xon/Xoff) keys should not be used for this purpose if 'flow'  is

set to 'xon/xoff',  because they interfere  with the correct  operation of  the

comms device driver flow  control.  When the 'Home'  key is pressed, an  'xoff'

will be sent automatically  when the device driver's  receive buffer fills  up,

and an 'xon' will be sent as it empties after the 'Home' key has been pressed a

second time to unfreeze the screen.  All other keys are ignored when the screen

is frozen.  The status line indicates when the emulator is in this state.

Information which scrolls off the top of the screen is not in fact lost, but is

stored in an "extended display buffer",  which can be examined by pressing  the

'PgUp' key.  The extended display buffer can contain a number of screenfulls of

data, and the 'PgUp' and 'PgDn' keys  can be used to range freely through  this

data.  If  any  other key  is  pressed while  the  extended display  buffer  is

visible, the current screen contents are redisplayed and the keystroke is  sent

to the host.   The 'PgUp' and  'PgDn' keys may  be used even  when the host  is

still sending data.  If  Xon/Xoff flow control  is in effect,  no data will  be

lost.

The following VT102 features are not implemented:

- Smooth scrolling

- 132-column mode

- Alternate character ROM

- LED lamps

The VT102 keyboard autorepeat mode is always enabled.

When in connect mode, typing the  escape character (Control-]) followed by a  ?

for help will display a "pop-up" help window, indicating the options available.

These options are detailed in  the section on the  connect command.  If ^]c  is

typed to close the  connection, the screen  is restored to  its state when  the

'connect' command was issued.  A subsequent 'connect' will re-display the VT102

screen.

The control codes and  escape sequences recognised by  the VT102 emulation  are

listed below.  For full details of  the effects of these codes, please  consult

the VT102 manual.

ENQ           5     Send answerback message "OS/2 Kermit"

BEL           7     Sound beep

BS            8     Cursor left

TAB           9     Cursor to next tab stop

LF            10    Cursor down

VT            11    As LF

FF            12    As LF

CR            13    Cursor to left margin

SO            14    Select G1 character set

SI            15    Select G0 character set

CAN           24    Cancel escape sequence

SUB           26    As CAN

ESC           26    See below

Others              Ignored

ESC 7         Save cursor position

ESC 8         Restore cursor position

ESC D         Index

ESC E         Next line

ESC H         Set tab at current column

ESC M         Reverse index

ESC Z         Identify terminal

ESC c         Reset

ESC =         Enter application keypad mode

ESC >         Exit application keypad mode

ESC # 3       Double height and width emulation - top half of line

ESC # 4       Double height and width emulation - bottom half of line

ESC # 5       Single height and width

ESC # 6       Single height and double width emulation

ESC # 8       Screen alignment display

ESC ( g       G0 designator - g = A,B or 0 only

ESC ) g       G1 designator - g = A,B or 0 only

ESC [ Pn A    Cursor up

ESC [ Pn B    Cursor down

ESC [ Pn C    Cursor right

ESC [ Pn D    Cursor left

ESC [ Pl ;Pc H      Direct cursor address

ESC [ Pl ;Pc f      Direct cursor address

ESC [ Pn c    Identify report - response is ESC [ ? 6 ; 2 c

ESC [ 3 g     Clear all tabs

ESC [ 0 g     Clear tabs at current column

ESC [ ? Pn h  Set DEC private mode - modes supported as shown below

ESC [ ? Pn l  Reset DEC private mode - modes supported as shown below

mode no.        mode            set           reset

1           Cursor key      Application   Cursor

2           ANSI/VT52       N/A           VT52

5           Screen          Reverse       Normal

6           Origin          Relative      Absolute

7           Wraparound      On            Off

ESC [ Pn h    Set mode - modes supported as shown below

ESC [ Pn l    Reset mode - modes supported as shown below

mode no.        mode            set           reset

2           Keyboard lock   On            Off

4           Insert          Insert        Replace

20           Newline         CR LF         CR

ESC Pn i      Printer/screen on/off - 4 to 7 supported

ESC [ 5 n     Status report

ESC [ 6 n     Cursor position report

ESC [ Pn x    Request terminal parameter

ESC [ Pn ;Pn r      Set top and bottom margins

ESC [ 0 J     Erase to end of screen

ESC [ 1 J     Erase from beginning of screen

ESC [ 2 J     Erase all of screen

ESC [ 0 K     Erase to end of line

ESC [ 1 K     Erase from beginning of line

ESC [ 2 K     Erase all of line

ESC [ Pn L    Insert blank lines

ESC [ Pn M    Delete lines

ESC [ Pn @    Insert blank characters

ESC [ Pn P    Delete characters

ESC [ Ps ;Ps ; ..;Ps m                                                         Character attributes or

ESC [ Ps ;Ps ; ..;Ps }                                                         Character attributes, as below:

0       Default settings

1       High intensity

4       Underline

5       Blink

7       Reverse

8       Invisible

30-37   sets foreground colour to be as shown

30      black

31      red

32      green

33      yellow

34      blue

35      magenta

36      cyan

37      white

40-47   sets background colour to be as shown

40      black

41      red

42      green

43      yellow

44      blue

45      magenta

46      cyan

47      white

Note that the  default character  set for  both G0  and G1  is 'A',  ie the  UK

character set.

The following escape  sequences are recognised  when the emulator  is put  into

VT52 mode by receiving the sequence ESC [ ? 2 l.

ESC A       Cursor up

ESC B       Cursor down

ESC C       Cursor right

ESC D       Cursor leftup

ESC F       Enter graphics mode

ESC G       Exit graphics mode

ESC H       Cursor to home

ESC I       Reverse line feed

ESC J       Erase to end of screen

ESC K       Erase to end of line

ESC Y l c   Direct cursor address

ESC Z       Identify

ESC =       Enter application keypad mode

ESC >       Exit application keypad mode

ESC <       Enter ANSI mode

The escape sequences below are accepted but ignored.

ESC O x     where x is any character

ESC ? x     where x is any character

ESC [ Pn q  Load LEDs

7 C-KERMIT RESTRICTIONS AND KNOWN BUGS

If you find any bugs which are not documented here, please send details to  the

author:

Dr C J Adie

Edinburgh University Computing Service

Main Library

George Square

Edinburgh

EH8 9LJ

United Kingdom

or   (preferably)   by   electronic   mail    on   the   Janet   network    to:

C.J.Adie@UK.AC.EDINBURGH

1. Server breakout:  There is  no way  of stopping  server operation  from  the

keyboard, short of Control-Break.

2. Debugging log: There is  very little debugging  information logged from  the

OS/2-specific parts of the program (it was developed using Codeview).

3. Set Delay: This should default to zero.  Currently it has no effect.

4. Dial: The DIAL appears not to work.  The author has no convenient access  to

a dial-out modem, so is not able to test it out properly.  It is  impossible

to terminate the dialling process short of pressing Control-Break.

5. Keyboard handling: In interactive command mode (as opposed to connect mode),

the keyboard has  only the  natural key-to-ascii-code  mapping.  This  means

that (for instance) the Del key does not  produce code 127, but a code of  0

followed by 224.   A keyboard remapping  feature (perhaps incorporating  key

macros) would be useful in many ways.

In connect mode, it would be more logical to use the Scroll Lock key as  the

equivalent of the  VT102 No Scroll  key, but this  means writing a  keyboard

monitor, as Scroll Lock is not placed in the keyboard input buffer.

6. Terminal emulation:  If  the host  sends  the  escape sequence  to  put  the

terminal into  132-column  mode, and  subsequently  sends data  which  would

appear in the rightmost 52  columns, this may mess  up the existing data  on

the screen.  Really the emulator should ignore any data for these columns.

7. Answerback: We should have a 'set terminal answerback ...' command to let us

change the VT102 answerback message.

8. Character set: We  should have a  'set terminal characterset'  to allow  the

default character set to be  either the UK or the  US set.  At present,  the

default is UK.

9. File type: The way Control-Z is handled could be better.  There should be no

'set file type  {binary, text}';  instead we want  a 'set  ctrlz {on,  off}'

rather like  Kermit-MS.  A  better  display of  progress  of a  transfer  is

needed, and  the interrupt  keystrokes should  conform to  the usual  Kermit

conventions.

10. Hangup: There  should be  a 'hangup'  command  (as well  as the  ^]h  escape

sequence), which would drop DTR  and RTS, thus hanging  up a call through  a

modem.

11. Login Scripts: The  present login  scripts implementation  follows the  Unix

conventions of UUCP's 'L.sys'  file, rather than  the normal Kermit  'INPUT/

OUTPUT' style.

APPENDIX A

INVOKING C-KERMIT FROM ANOTHER PROGRAM

If you are writing a communications program and wish to incorporate the  Kermit

protocol within it, one way is to use the OS/2 function call DosExecPgm to call

up C-Kermit.  You would supply  the instructions for Kermit using  command-line

options, and Kermit would do the transfer, returning back to your program  when

it had finished.

The only problem with this  scenario is that you  might already have opened  up

the COM port within your program, so that  when Kermit tries to do the same  it

gets an error code back  from DosOpen.  The -u  command line option gets  round

this problem.  It uses  the fact that  a child process  inherits the open  file

handles of its parent.  -u takes one  numeric parameter which is the handle  of

the COM port in question, and it must occur in front of any other  command-line

parameter which accesses the COM port.   The following is a complete C  program

written using  the Microsoft  C compiler  version 5.1  and the  Microsoft  OS/2

Software Development Toolkit, which illustrates how to use the -u  command-line

option.

#define INCL_BASE

#include <os2.h>

/*

* Example of how to use the C-Kermit -u option to invoke

* Kermit from another program under OS/2.

*/

main(int argc, char *argv[]) {

HFILE ttyfd;

USHORT action;

int err,i;

char failname[80];

char args[80];

RESULTCODES res;

struct dcb {   /* Device control block */

USHORT write_timeout;

USHORT read_timeout;

BYTE flags1, flags2, flags3;

BYTE error_replacement;

BYTE break_replacement;

BYTE xon_char;

BYTE xoff_char;

} ttydcb;

/*** Open a file ***/

if (err=DosOpen(argv[1],&ttyfd,&action,0L,0,1,0x0012,0L)) {

printf("Error %d opening %s\n",err,argv[1]);

exit(1);

}

if (err=DosDevIOCtl(&ttydcb,NULL,0x0073,1,ttyfd)) {

printf("Error %d from IOCTL on %s\n",err,argv[1]);

exit(1);

}

ttydcb.flags3 &= 0xF9;

ttydcb.flags3 |= 0x04; /* Read "some" data from line */

DosDevIOCtl(NULL,&ttydcb,0x0053,1,ttyfd);

/*** Call kermit ***/

strcpy(args,"ckoker");

i = strlen(args);

args[i++]=0;

sprintf(&args[i],"-u %d -q -s test.c",ttyfd);

i += strlen(&args[i]);

args[i++]=0;

args[i++]=0;

if (err=DosExecPgm(failname,80,EXEC_SYNC,args,NULL,&res,

"KERMIT.EXE")) {

printf("Error %d executing Kermit\n",err);

exit(1);

}

/*** Print out return code ***/

printf("Termination code %d\n",res.codeTerminate);

printf("Result code %d\n",res.codeResult);

/*** Close the file ***/

if (err=DosClose(ttyfd)) {

printf("Error %d closing %s\n",err,argv[1]);

}

}

INDEX

Dialout Modems  1

Emergency Exit  1

File Warning  1

Hayes Modem  1

IBM  1

Modems  1

Warning  1

TABLE OF CONTENTS

1 Introduction                                                              1

1.1 Acknowledgements                                                      1

1.2 OS/2 Kermit Capabilities                                              1

1.3 Requirements                                                          1

1.4 The Serial Port                                                       1

1.5 Emergency Exit                                                        1

2 The OS/2 File System                                                      1

2.1 File Specifications                                                   1

2.2 File Formats                                                          1

3 File Transfer                                                             1

4 Command Line Operation                                                    1

4.1 Command Line Options                                                  1

4.2 Command Line Examples                                                 1

4.3 Exit Status Codes                                                     1

5 Interactive Operation                                                     1

5.1 The 'send' command                                                    1

5.2 The 'receive' command                                                 1

5.3 The 'get' command                                                     1

5.4 The 'server' command                                                  1

5.5 The 'remote', 'bye', and 'finish' commands                            1

5.6 The 'log' and 'close' commands                                        1

5.7 Local File Management Commands                                        1

5.8 The 'set' Command                                                     1

5.9 The 'show' Command                                                    1

5.10 The 'statistics' Command                                             1

5.11 The 'take' and 'echo' Commands                                       1

5.12 The 'connect' Command                                                1

5.13 The 'dial' command                                                   1

5.14 The 'script' Command                                                 1

5.15 The 'help' Command                                                   1

5.16 The 'exit' and 'quit' Commands                                       1

6 Terminal Emulation                                                        1

7 C-Kermit Restrictions and Known Bugs                                      1

A Invoking C-Kermit from Another Program                                 1