The following VT100 escape sequences are performed by MacKermit 0.97(57) VT100

emulation:

Cursor Movement Commands:

Cursor Up                           ESC[PnA

Cursor Down                         ESC[PnB

Cursor Forward (right)              ESC[PnC

Cursor Backward (left)              ESC[PnD

Direct Cursor Addressing            ESC[Pl;PcH or ESC[Pl;Pcf

Index                               ESC D

New Line                            ESC E

Reverse Index                       ESC M

Save Cursor Attributes              ESC 7

Restore Cursor Attributes           ESC 8

Character Attributes:

ESC[Ps m

where Ps refers to a selective parameter as follows:

0 or none               All Attributes Off

1                       Bold on

4                       Underscore on

2, 3, 5, 6              Italics on (VT100 uses Blink)

7                       Reverse Video on

21, 22                  Bold off

24                      Underline off

25                      Italics off

27                      Reverse Video off

Erasing:

From Cursor to End of Line          ESC[K or ESC[0K

From Beginning of Line to Cursor    ESC[1K

Entire Line containing Cursor       ESC[2K

From Cursor to End of Screen        ESC[J or ESC[0J

From Beginning of Screen to Cursor  ESC[1J

Entire Screen                       ESC[2J

Character Sets:

The G0 and G1 character sets are designated with a Select sequence

followed by a designator letter.

Select (locking shift)

G0		ESC (

G1		ESC )

Character Set               	Designator   G1 designator

United States (USASCII)     	B

Special graphics characters 	0

and line drawing set

DEC Technical set			>

[Alternate Set] (USASCII)   	1

[Alternate Set] (USASCII)   	2

and special graphics

To select national character sets, the sequence "ESC [ ? 4 2 h"

(with no spaces).  National character set designators::

Dutch				4

Finnish				C or 5

French				R

French Canadian			9 or Q

German				K

Italian				Y

Norwegian/Danish			` or E or 6

Portuguese		(sequence:)	ESC % ( 6

Spanish				Z

Swedish				7 or H

Swiss				=

United Kingdom (UK)			A

United States (USASCII)		B

To select ISO 8859 international character sets, substitute "l" for "h"

in the above sequence.  ISO 8859 character set designators:

Latin 1				A

Scrolling Region:

ESC[Pt;Pb r

Pt is the number of the line of the top of the scrolling region; Pb is the

number of the bottom line of the scroling region and must be greater than

Pt.

Tab Stops:

Set tab at current position                 ESC H

Clear tab as current position               ESC[g or ESC[0g

Clear all tabs                              ESC[3g

Modes:

To Set                  To Reset

Mode Name           Mode      Sequence      Mode      Sequence

Insert/Replace      Insert    ESC[4h        Replace   ESC[4l

LineFeed/NewLine    NewLine   ESC[20h       LineFeed  ESC[20l

Screen Mode         Reverse   ESC[?5h       Normal    ESC[?5l

Origin Mode         Relative  ESC[?6h       Absolute  ESC[?6l

Wraparound          On        ESC[?7h       Off       ESC[?7l

Auto Repeat         On        ESC[?8h       Off       ESC[?8l

Reports:

Cursor Position Report

Invoked by          ESC[6n

Response            ESC[Pl;PcR

Status Report

Invoked by          ESC[5n

Response            ESC[0n  (terminal OK)

What Are You?

Invoked by          ESC[c or ESC[0c

Response            ESC[?1;2c  (VT100 with AVO, Advanced Video Option)

Printer Status Report

Invoked by          ESC[?15n

Response            ESC[?13n  (no printer)

Editing:

Insertion Mode: Enter       ESC[4h

Exit (Replacement Mode)     ESC[4l

Delete Character            ESC[PnP

Delete Line                 ESC[PnM

Insert Line                 ESC[PnL

Modes:

To Set                  To Reset

Mode Name           Mode      Sequence      Mode      Sequence

Cursor Key Mode     Application ESC[?1h     Cursor    ESC[?1l

Scrolling Mode      Smooth      ESC[?4h     Jump      ESC[?4l

Keypad Mode         Application ESC=        Numeric   ESC[>

Print Control Sequences:

Auto Print Mode: Enter              ESC[?5i

Exit               ESC[?4i

Printer Controller mode: Enter      ESC[5i

Exit       ESC[4i

Heath ANSI Control Sequences:

To Set		 		   To Reset

Mode		Sequence	Mode			Sequence

Block Cursor	ESC[>4h		Underline Cursor	ESC[>4l

Cursor Shown	ESC[>5h		Cursor Hidden		ESC[>5l

The following sequences are NOT performed by MacKermit 0.97(57) VT100 emulation:

Line Size:

Change this line to double-height top half          ESC#3

Change this line to double-height bottom half       ESC#4

Change this line to single-width single-height      ESC#5

Change this line to double-width single-height      ESC#6

Programmable LEDS:

ESC[Ps q    set LED on or off

Modes:

To Set                  To Reset

Mode Name           Mode      Sequence      Mode      Sequence

ANSI/VT52 Mode      ANSI        N/A         VT52      ESC[?2l

Column Mode         132 Col     ESC[?3h     80 Col    ESC[?3l

Interlace           On          ESC[?9h     Off       ESC[?9l

Print Termination   Form Feed   ESC[18h     None      ESC[18l

Print Extent        Full Page   ESC[19h     Window    ESC[19l

Print Control Sequences:

Print Line                          ESC[?1i

Print Page                          ESC[i

Report Terminal Parameters:

ESC[P1 n

Request terminal parameters

Reset:

ESC c

Confidence Tests:

ESC#8

ESC[2;Ps y

All VT52 mode commands:

Cursor Up                           ESC A

Cursor Down                         ESC B

Cursor Right                        ESC C

Cursor Left                         ESC D

Select Special Graphics             ESC F

Select ASCII Characters             ESC G

Cursor to Home                      ESC H

Reverse Line Feed                   ESC I

Erase to End of Screen              ESC J

Erase to End of Line                ESC K

Direct Cursor Address               ESC Y l c

Identify                            ESC Z           response:  ESC / Z

Enter Alternate Keypad Mode         ESC =

Exit Alternate Keypad Mode          ESC >

Enter ANSI mode                     ESC <

Enter Printer Controller mode       ESC W

Exit Printer Controller Mode        ESC X

Enter Auto Print Mode               ESC ?

Exit Auto Print Mode                ESC _

Print Line                          ESC V

Print Page                          ESC ]