File DIS_PKT.DOC						     3 Nov 1991

Joe R. Doupnik

Utah State University

Logan Utah 84322

jrd@cc.usu.edu

(801) 750-2982 voice

(801) 750-2992 fax

DIS_PKT.DOS is a "shim" to convert between an NDIS network board driver and an

application that wants to talk to a packet driver.  It shares the board with

other NDIS users.

Packet-Driver-flavored applications		NDIS-flavored applications

||					||

-------------				||

|  DIS_PKT  |				||

-------------				||

||					||

------------------------------------------

|           main module			  |

|   NDIS    ........................      |

|	    board specific driver(s)	  |

-------------------------------------------

||

--------------------------------

| Ethernet/Token Ring board(s)	|

--------------------------------

||

=================================== Ethernet/Token Ring wire

Material added to version 1.09 (since v1.07) -

o Support for using old Novell 802.3 packets on the wire while using Type

8137 packets to the application. The BYU Packet Driver Novell shell

kit requires Type 8137 packets be used by the shell. The keyword "NOVELL="

in the section [pktdrv] determines whether or not conversion occurs. If a

y is placed after the NOVELL= phrase then conversion occurs:

novell = y		(case independent)

This option provides the same feature as the "-n" option of a regular

Packet Driver from the Clarkson collection.

Conversion does not occur if the right hand side is not a y, nor if the

entire line is omitted.

Conversion occurs only if the NDIS driver uses Ethernet 802.3 packets; this

also means dis_pkt provides a Packet Driver Class 1 interface to apps.

o Support for Ethernet 802.3 with 802.2 headers and Token Ring 802.5 NDIS

drivers, as Packet Driver Class 11 and 3, respectively. The NOVELL= option

does not apply to these boards. Dan Lanciani of Harvard added this part.

First sample PROTOCOL.INI file:

[protocol manager]

drivername = PROTMAN$

[pktdrv]			<-- name of this driver entry

drivername = pktdrv$		<-- formal driver name

bindings = wd8003xmac		<-- use your board's NDIS driver here

intvec = 0x60			<-- Packet Driver Int, 60h..7fh

chainvec = 0x66		<-- daisy chained Pkt Drvr Int, unused

novell = y			<-- Optional, if present and y(es) then

convert between old Novell 802.3 pkts

on the wire and Type 8137 for the app.

Omitting this line or using any other

response turns off the conversion;

default is no conversion.

[attiso]

drivername = ATTISO$		<-- Another NDIS client

bindings = wd8003xmac		<-- bound to the same harware driver

nsess = 5

ncmds = 14

use_emm = n

Western Digital EtherCard PLUS Family Adapter	<-- Ethernet board

[wd8003xmac]					<-- its ndis driver

drivername = MACWD$

irq = 7

ramaddress = 0xCA00

iobase = 0x280

receivebufsize = 1536	<-- make this a full Ethernet pkt

Sample section of CONFIG.SYS (StarGROUP material is not required):

device=c:\lanman\protman.sys /i:c:\lanman	<-- must be first

device=c:\lanman\macwd.dos			<-- WD8003E driver

device=c:\lanman\dis_pkt.dos			<-- Pkt Driver (this program)

device=c:\lanman.dos\drivers\attload.dos /Y	<-- StarGROUP NDIS

device=c:\lanman.dos\drivers\attiso\attiso.dos	<-- StarGROUP NDIS

device=c:\qemm\loadhi.sys /r:1 e:\pctcp\ifcust.sys  <-- PC/TCP stuff

device=c:\qemm\loadhi.sys /r:4 e:\pctcp\ipcust.sys  <-- etc

A second, more elaborate example, with names easier to type.  We start with

file PROTOCOL.INI.  Note that semicolons start comment lines.

; This is a sample protocol.ini file listing three Ethernet boards:

;	attcsma.dos is an AT&T StarLAN 10 EN100

;	elnkii.dos  is a 3Com 3C503

;	wd8003.dos  is a Western Digital WD8003E

; Only one board will be selected but the other two are present.

[protocol manager]

drivername = PROTMAN$

; Packet Driver protocol users tie in here

[pktdrv]

drivername = pktdrv$

bindings = attcsma

;	bindings= elnkii

;	bindings = wd8003

intvec = 0x60

;	chainvec = 0x66		; chaining to another Packet Driver is unused

novell = no		; do not convert packet types this time

; AT&T StarGROUP protocol stack ties in here via name ATTISO$

[attiso]

drivername = ATTISO$

bindings = attcsma

;	bindings = elnkii

;	bindings = wd8003

nsess = 5

ncmds = 14

use_emm = n

;Western Digital EtherCard PLUS Family Adapter, WD8003E in this case

[wd8003]

drivername = MACWD$

irq = 7

ramaddress = 0xCA00

iobase = 0x280

receivebufsize = 1536

;	maxtransmits = 6

;	receivebuffers = 6

;	receivechains = 6

; 3Com Etherlink II, 3C503

[elnkii]

drivername = ELNKII$

ioaddress = 0x350

interrupt = 5

transceiver = onboard

maxtransmits = 12

xmitbufs = 1

; AT&T StarLAN 10 EN100

[attcsma]

drivername = ATTCSMA$

board_type = 2

irq = 2

ioaddr = 0x360

daram = 0xD000

; End of file protocol.ini

Fragment of config.sys for the second example. Note three .dos board drivers.

device=c:\system\ramdrive.sys 1024 512 128 /E

device=c:\lanman\protman.sys /i:c:\lanman.dos\drivers\star10en

device=c:\lanman\attcsma.dos

device=c:\lanman\elnkii.dos

device=c:\lanman\macwd.dos

device=c:\lanman\dis_pkt.dos

device=c:\lanman.dos\drivers\attload.dos /Y

device=c:\lanman.dos\drivers\attiso\attiso.dos

device=c:\qemm\loadhi.sys /r:1 e:\pctcp\ifcust.sys

device=c:\qemm\loadhi.sys /r:4 e:\pctcp\ipcust.sys

device=c:\qemm\loadhi.sys /r:4 c:\netdev.sys

shell=c:\command.com  /p /e:800

It seems to be necessary to run NETBIND.EXE to get all this to be active.

Some Lan Manger network startup routines run Netbind implicitly.

Copyright notice and disclaimer from the beginning of the source code

file dis_pkt.asm:

; DIS_PKT.ASM - Adapter provides Packet Driver v1.09 interface over NDIS.

; Version 1.07  18 May 1991  by Joe R. Doupnik, Utah State Univ.

; Version 1.08  9 Aug 1991 by Dan Lanciani, ddl@harvard.harvard.edu

; Version 1.09  3 Nov 1991 by Joe R. Doupnik, Utah State Univ.

; Copyright (C) 1988 - 1991 FTP Software, Inc.

This unmodified source file and its executable form may be used and

redistributed freely.  The source may be modified, and the source or

executable versions built from the modified source may be used and

redistributed, provided that this notice and the copyright displayed by the

exectuable remain intact, and provided that the executable displays an

additional message indicating that it has been modified, and by whom.

FTP Software Inc. releases this software "as is", with no express or implied

warranty, including, but not limited to, the implied warranties of

merchantability and fitness for a particular purpose.

USE AT YOUR OWN RISK.

To build, using Microsoft MASM 5 or later, LINK 3.64 or later, and EXE2BIN:

masm dis_pkt;

link dis_pkt;

exe2bin dis_pkt.exe dis_pkt.dos

del dis_pkt.exe

del dis_pkt.obj

Here is an excerpt from the formal NDIS v2 specification document by 3Com

and Microsoft concerning what goes into the PROTOCOL.INI file.

PROTOCOL.INI

The PROTOCOL.INI file stores configuration and binding information for all the

protocol and MAC modules in the system.  The file uses the same general format

as the LANMAN.INI file.  It consists of a series of named sections, where the

section name is in fact the module name from a module characteristics table.

Below the bracketed module name is a set of configuration settings for the

module in name=value format.  For example:

[MYNetBIOS]

Drivername = NetBIOS$

Bindings = ETHERCARD

MaxNCBs = 16

MaxSessions = 32

MaxNames = 16

The rules for PROTOCOL.INI contents are:

o Bracketed module name.  Must be the name of a protocol or MAC module, e.g.

[MYNetBIOS].  This is the name of the module as defined in that module's

characteristics table.  The name must be 15 characters or less (not

counting the brackets).  Mixed case may be used but the Protocol Manager

will convert it to uppercase when it reads the file into memory.

o Drivername = <device driver name>.  This parameter is required for all

device driver modules.  It defines the name of the OS/2 or DOS device

driver that the module is contained in.  Note that a single device driver

name may be mentioned by several sections of the PROTOCOL.INI file, if the

driver contains multiple logical modules.  The Drivername parameter is the

recommended method by which a module searches for its module section in the

PROTOCOL.INI file to get its configuration parameters.  This allows the

module to find all relevant module sections based on a single name

intrinisic to the module independent of the particular bracketed module

name used in the PROTOCOL.INI file.  This keyword is also required for DOS

dynamic modules like TSRs or transient application modules.  Although there

is no driver name instrinsically assigned to such modules it is required

that a unique name be assigned to this keyword for such modules anyway.  In

this way the same search mechanism used by device drivers can be used by

dynamic DOS modules to find their relevant module sections in PROTOCOL.INI.

o Bindings = <module name> | <module name>,<module name>, . . .

This parameter is optional for protocol modules.  It is not valid for MAC

modules.  If present, it is used by the protocol module to determine what

MAC modules it will ask to bind to.  (In other words, changing this

parameter in the PROTOCOL.INI file can reconfigure a protocol to bind to a

different MAC.).  The Bindings parameter may be omitted if the protocol

driver software is preconfigured to bind to a particular MAC, or if the

system will only contain one MAC and one static protocol module.  In the

latter case (only in static mode), the Protocol Manager by default will ask

the one static protocol to bind to the one MAC.

o Other keywords and parameters.  Any other keyword=value statements are

module specific.  Keyword names must be 15 characters or less.  They may be

mixed case but are converted to uppercase when read by the Protocol

Manager.  Note that keyword names are unique within the scope of each

<module name> section and can appear within the section in any order.

o Whitespace around the equals sign is not significant, nor is trailing white

space on the line.  Except for this leading and trailing white space, all

other characters of the value string are taken verbatim.

o A list of 0 or more parameters can appear to the right of the equals sign.

If there are no parameters the equals sign can be optionally omitted.  A

parameter is terminated by a space, tab, comma, or semicolon.  No

parameters are interpreted by the Protocol Manager.

o A parameter can either be up to a 31-bit signed numeric value or a string

of any length.

o A numeric parameter can be expressed either in decimal or hexadecimal

format.  All numeric parameters must start with the characters '0' through

'9' or by a + or - followed by the '0' to'9' character.  A hexadecimal

parameter must start with '0x' or '0X' and use valid hexadecimal digits.  A

non-hexadecimal numeric parameter is treated as decimal integer.  A

parameter not surrounded by quotes and starting with 0 to 9 or + and -

followed by 0 to 9 will be assumed to be a numeric parameter.

o A string is a parameter which either starts with a non-numeric character or

is surrounded with quotes ("....").  The string is preserved in the memory

image as it appears in PROTOCOL.INI.

o A line starting with a semicolon in column 1 is a comment and is ignored.

Blank lines are ignored too.

o Lines may be as long as required.  Continuation lines are not supported.

Lines end with CR LF.

o Tabs, formfeeds, and spaces are considered to be white space.

The Protocol Manager supports an optional section with optional keywords

defined below:

[PROTMAN]

Drivername = PROTMAN$

Dynamic = YES or NO

PRIORITY = prot1, prot2, ...

Bindstatus = YES or NO

The bracketed module name can be any valid name as long as it is unique within

this PROTOCOL.INI.  Drivername is required and must be assigned PROTMAN$,

identifying the section as belonging to the Protocol Manager.  None of the

entries are case-sensitive.

The DYNAMIC keyword is optional.  It defaults to NO if not present.  If set to

NO, the Protocol Manager operates only in the static mode and does not support

dynamic protocol drivers.  If set to YES, the Protocol Manager operates in the

dynamic mode and supports both static and dynamic binding.

The PRIORITY keyword is optional.  If absent, then the VECTOR uses default

demultiplexing priority if multiple protocol drivers are bound to the same MAC

(see Vector Demultiplexing in Chapter 7).  If present, the parameters on the

right-hand side are presumed to be a list of protocol module names, highest

priority first.  The VECTOR prioritizes protocol drivers for demultiplexing

(if necessary) according to their order in the list, and packets are offered

to the first protocol driver listed first.  Protocol drivers not listed are

assigned default priority AFTER those listed.  It is not necessary that a

protocol driver ever bind for it to be listed here.

The BINDSTATUS keyword is optional.  If absent, then the BindStatus command is

not supported by the Protocol Manager.  If set to YES, then BindStatus is

supported by the Protocol Manager.  The default disable condition is a memory

optimization feature primarily for DOS environments.

End of file DIS_PKT.DOC