File MSTZ10.DOC							28 Jan 1991

DRAFT								DRAFT

Documentation for MSKERMIT Version 3.0 for the Heath/Zenith-100.

This short document only describes special features for this specific

implementation for the Z-100.  For general Kermit information, please refer to

KERMIT.DOC, MSKERMIT.DOC and MSKERMIT.HLP etc.

This issue corrects several serious problems with buffer overruns when using

large packets and/or windowing.  It also includes improvements in the areas of

screen writing speed and handling of escape codes for the S-100 ports.

Further, the screen is now saved, when leaving terminal mode and restored upon

reentry of terminal mode.  This is particularly nice for the use of HELP or

SHIFT-HELP.  A one screen (26 line) scrolling function has also been added.

Work is slowly going on to implement VT100 terminal emulation, full multipage

scrolling and colors.  It takes time, however.  I hope I will have it finished

before all Z-100s but mine are scrapped.

NOTE!!          Using the US Robotics S-100 ports, at 1200 baud only,

this Kermit implementation may lose the first character

on the line if run at the original lower clock frequency

of the H/Z-100.  If run at 7 MHz or above there should be

no problem.  The easy fix is to tell the host to use one

null character at the beginnig of each new line.  (For

VAX VMS use "set terminal/lf=(1)")

Implements:

1. Set terminal Heath-19, set terminal None

2. Set port com1/com2/com3/com4 or 1/2/3/4 or J1/J2 or A/B (Note com1=1=J2=B,

com2=2=J1=A)

3. Support for 2 US Robotics S-100 modems at ports com3/com4 (com3 port

address 0020H, com4 port address 002EH)

4. Buffered port input and xon/xoff handshaking in terminal mode also for the

S-100 ports (to some extent).

5. Automatic sensing of presence of S-100 modem boards and automatic sensing

of carrier for the boards present.

6. Automatic selection of port

Explanations:

1. The Heath/Zenith-100 terminal inherently supports the VT-52 and the

Heath-19 terminal rules.  The current keypad mode is indicated by N or A on

the mode line right after the terminal emulation info.

Separate terminal drivers like zansi.dvd may be used through the command

"ctty ansi" before invoking Kermit which then has to be left in native

terminal mode (set terminal none).

Note, however, that screen writing is done through DOS in this mode and it

is therefore very slow, particularly when performed via ANSI.

The VT52 keys PF1 (Gold) to PF4 are mapped onto the top four keys of the

shifted keypad, the VT52 - key onto the shifted down arrow and the VT52,

key onto the shifted minus key.  This makes the keypad (numeric and

application mode) close in function to the VT52 keypad.

2. The H/Z-100 computer has two serial ports designated J1 or A and J2 or B.

Because the B port is of the DTE type and is the primary modem port, port B

or J2 is designated com1 or port 1.

Use the command set port ? to get help.  The basics are as follows:

- set port com1, set port 1, set port B, set port J2

All activate communication over J2 at the current (as set with cofigur)

AUX baud rate and parity.

- set port com2, set port 2, set port A, set port J1

All activate communication over J1 at the current AUX baud rate and

parity.

- set port com3, set port 3          (address 0020H)

All activate communication over the first S-100 port at the current baud

rate and parity. Default is 1200 baud.

- set port com4, set port 4          (address 002EH)

All activate communication over the second S-100 port at the current

baud rate and parity. Default is 1200 baud.

3. The US Robotics S-100 modem is not interrupt driven, so the port has to be

polled and care has to be taken not to loose characters when doing other

things like screen output and disk transactions.  -- See 4. below.

4. When, in terminal mode, the escape sequencies ESC-E or ESC-J are sent to

the screen or disk writing is done, characters will be lost unless special

steps are taken by the software.

The two special escape sequencies mentioned are broken down into smaller

steps for the screen handler and the port is polled in between these steps.

(ESC-E = ESC-H + ESC-J, ESC-J = ESC-K + possibly CR + a number of LF +

ESC-K combinations)

During disk capture disk writing the host is told to stop sending (xoff),

a delay to cover for transmission delays follows, then the disk capture

buffer is written after which the host is told to continue (xon). The port

is polled from the sending of xoff until the end of the delay and the

received characters are put in a buffer. The disk capture buffer is only

128 bytes long, so there will be a pause for every 128 bytes.

The pause duration may be varied by setting the receive timeout parameter

(set receive timeout) to some value other than the default.  0 results in

no delay, the default of 5 gives less than half a second and the maximum of

94 a few seconds.  I have adjusted the timing to work reliably over normal

telephone lines with a VAX as a host.  You may be able to speed up by

setting the timeout value as low as 1.  On the other hand, if computers

further away in a network are hosting or if satellite communications are

involved you may have to adjust the timing upwards.

5. The testing for presence of S-100 modems and carriers for these takes

place when Kermit is started and a message is issued to inform the user.

I have found the automatic sensing of S-100 modems to work reliably.

There is a possibility, however, that other boards at the addresses in

question will interfere with this function.

6. Default port is in priority sequence:

- Port 3 if carrier detected there

- Port 4 if carrier detected there

- The port designated AUX

7. Use the command show key to find out exactly how the keys are mapped.

For your conveniance, here are the most important ones.

- Most function and special keys provide normal Z-100 escape sequences

- F12   toggles the terminal printer on/off.In native mode use CTRL-P.

- SF12  (dump screen to printer is not available to Kermit)

- SF11  dumps screen to a file (default kermit.scn)

- F11   enters video memory screen scrolling, 26 lines; use Back Space and

Line Feed keys to scroll and Return key to "freeze" position; any

other key to restore

- HELP  displays the connect help message

- SHELP displays the status message

- The shifted top 4 numeric keypad keys) provide VT52 PF1 (gold) to PF4

8. Known shortfalls, bugs and potential problems

- Windowing does not work well for the S-100 modems, COM3 and COM4,

obviously because these modems are not interrupt driven.  The windowing

routines eat cpu time in chunks too big to allow proper port polling

(unless the code in the 'generic' source files is modified).

- The screen sometimes 'tears' when entering terminal mode (defect in

atsclr).  This is cosmetic only.

- Show modem only partially supported due to hardware limitations.  Avoid

untested and unproven use of SHOW MODEM and WAIT \CD, \DSR.  Ports 1 and

2 support \CD and \DSR, ports 3 and 4 only \CD.

- VT52 type printer control is not supported.

- Set term none will set key off and set term Heath will set key on.

(Set term will not follow set key on/off)

- Screen scrolling, beyond F11 above, is not implemented.

- Certain character translation features are implemented. These features,

however, are completely untested at this time.

Good luck using Kermit

Bo Gedda