File MSVRB1.DOC							2 March 1991

DEC Rainbow 100 MS-DOS Kermit Version 3.10 Release Notes

By Robert Weiner (rweiner@watsun.cc.columbia.edu)

Many of the new features and bug fixes which are contained in DEC Rainbow 100

MS-Kermit 3.10 are ones which were made to the system independent

modules of MS-Kermit.  These changes are described in the 3.10 release nodes

by Joe Doupnik.

Most of the recent changes to the Rainbow specific modules were made by

Gary B. Stebbins and myself.  The following improvements have been made

to the DEC Rainbow 100 system specific modules:

1) Rainbow kermit now properly sets the modem control signals.

The HANGUP, SHOW MODEM and WAIT (for modem signal) commands now

work properly.

2) Connect mode hangup command (Ctrl-] h) now works properly.

3) Baud rate is now properly displayed in the STAT and SHOW MODEM commands.

4) Various improvements were made to the printer control mechanism.

5) A new print-screen/dump-screen character translation table has been

added which will properly display VT100 line drawing special

characters in a printable form in both the dump-screen output file

and on the printer.

6) Various speed enhancements were made to the screen handling code.

7) A non-terminated string in the SET KEY command caused "garbage" to be

displayed on the screen.  Now SET KEY displays only what it

is supposed to.

Building DEC Rainbow 100 MS-DOS Kermit From Its Source Code

===========================================================

There are numerous methods for obtaining copies of Kermit and its source

code.  Most of these methods will not be described here.

The computer at Columbia University which provides access, by anonymous FTP,

to the Kermit programs, source and documentation is "watsun.cc.columbia.edu".

On this machine, the official releases of DEC Rainbow 100 MS-Kermit may be

found in the ~kermit/a directory.  Newer versions, which are often test

versions under current development, may be found in the ~kermit/test directory.

It should be noted that the files in the ~kermit/test directory have a

tendency to change many times each week as improvements and corrections

are made to the source code.

The source code for DEC Rainbow 100 MS-Kermit is made up from the following

system independent (MSS*.*) and the Rainbow 100 system dependent modules

(MS?RB1.*):

MSSDEF.H,   MSSCMD.ASM, MSSCOM.ASM, MSSFIL.ASM, MSSKER.ASM,

MSSRCV.ASM, MSSSCP.ASM, MSSSEN.ASM, MSSSER.ASM, MSSSET.ASM,

MSSSHO.ASM, MSSTER.ASM, MSURB1.ASM, MSXRB1.ASM

To build MS-Kermit from these modules, a MS-DOS single line command such as

for %i in (ms*.asm) do masm %i;

may be used to assemble all the modules.  Then the command:

link @msvrb1.lnk

can be used to link the modules into the executable MSVRB1.EXE.

Sample DEC Rainbow 100 MSKERMIT.INI Initialization File

========================================================

The following is an extract of my kermit initialization file.  It

demonstrates the use of some of the nice options available in MS-Kermit

such as Sliding Windows, Long Packets and Attribute Packets:

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+

echo VT102 Keyboard Definitions Loading...\d13

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+

;		DEC RAINBOW 100 MSKERMIT 3.0 definitions

;		By Robert Weiner, Programming Plus (15-APR-90)

;		EMAIL:	rweiner@watsun.cc.columbia.edu

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+

;		First we set up some kermit options

; ---------------------------------------------------------------------+

set file type binary		; Send/Recv files identically

set incomplete keep		; Keep incomplete recvd files

set timer on			; Enable timer (timeout processing)

set attributes on		; Send/Recv File Attributes (Date/Time/Size)

set bell on			; Beep after send/recv

set file warning on		; Handle filename collisions

set display 8-bit		; Display chars as 8-bit

set term roll on		; Roll screen back if new chars recvd.

set send packet 2000		; Enable long packets for send

set receive packet 2000		; Enable long packets for recv

set window 2			; Enable 2 sliding windows

set block 3			; Long packets require 3-char CRC

set retry 10			; Allow 10 retries

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+

;		Now reset the default function keys as real VT102

;		Remap Kermit Function & Screen Rollback Keys

;			    SHIFT-DO = Screen Dump	(\Kdump)

;		     SHIFT-PREV/NEXT = Prev/Next Screen (\Kupscn,\Kdnscn)

;		      CTRL-PREV/NEXT = Prev/Next Line	(\Kupone,\Kdnone)

;		CTRL-SHIFT-PREV/NEXT = Top/Bottom	(\Khomscn,\Kendscn)

; ---------------------------------------------------------------------+

;					Make SHIFT-DO = \Kdump

set key \769	\Kdump

;					Make SHIFT-PREVSCREEN = \Kdnscn

set key \803	\Kdnscn

;					Make SHIFT-NEXTSCREEN = \Kupscn

set key \805	\Kupscn

;					Make CTRL-PREVSCREEN = \Kdnone

set key \1315	\Kdnone

;					Make CTRL-NEXTSCREEN = \Kupone

set key \1317	\Kupone

;					Make CTRL-SHIFT-PREVSCREEN = \Khomscn

set key \1827	\Khomscn

;					Make CTRL-SHIFT-NEXTSCREEN = \Kendscn

set key \1829	\Kendscn

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+

;		MAKE RAINBOW KEYS SEND RAINBOW ESC SEQUENCES

; ---------------------------------------------------------------------+

;					F4

set key \261	\o33\o133\o61\o64\o176

;					F6 - INTERRUPT

set key \263	\o33\o133\o61\o67\o176

;					F7 - RESUME

set key \265	\o33\o133\o61\o70\o176

;					F8 - CANCEL

set key \267	\o33\o133\o61\o71\o176

;					F9 - MAIN SCREEN

set key \269	\o33\o133\o62\o60\o176

;					F10 - EXIT

set key \271	\o33\o133\o62\o61\o176

;					F14 - ADDTNL OPTIONS

set key \273	\o33\o133\o62\o66\o176

;					HELP

set key \256	\o33\o133\o62\o70\o176

;					DO

set key \257	\o33\o133\o62\o71\o176

;					F17

set key \275	\o33\o133\o63\o61\o176

;					F18

set key \277	\o33\o133\o63\o62\o176

;					F19

set key \279	\o33\o133\o63\o63\o176

;					F20

set key \281	\o33\o133\o63\o64\o176

;					FIND

set key \283	\o33\o133\o61\o176

;					INSERTHERE

set key \285	\o33\o133\o62\o176

;					REMOVE

set key \287	\o33\o133\o63\o176

;					SELECT

set key \289	\o33\o133\o64\o176

;					PREVSCREEN

set key \291	\o33\o133\o65\o176

;					NEXTSCREEN

set key \293	\o33\o133\o66\o176

;					SHIFT. = .

set key \814	.

;					SHIFT, = ,

set key \812	,

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+

echo VT102 Keyboard Definitions Loaded\d13\d10

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+

; [End Of MSKERMIT.INI]

; ---------------------------------------------------------------------+

; ---------------------------------------------------------------------+